New (Keel Laid on or after 01/01/82, Panama)

| NAME OF SHIP: | PORT OF REGISTRY: | GROSS TONNAGE: |  | YEAR KEEL LAID: |
| --- | --- | --- | --- | --- |
| Call Letters: | Patent Number and Date of Expiry: |  |  |  |
| Details of persons requesting survey and to whom certificates are to be sent: |  |  |  |  |
| Owner/Master |  |  | Agents/Operators: |  |

Vessels status

# Extent of Survey:

# GR CLASS CLASSIFIED FOR STANDARD (GR CLASS)

# SURVEY REPORT FOR THE ISSUE OF A CARGO SHIP SAFETY CERTIFICATE

## Vessels under 500 GRT

This form must be kept on board and available for inspection by a Nominated surveyor or recognized organization at all times

Existing (Keel Laid before 01/01/82, Panama)

Yes

No

N/A

Initial Survey:_

Periodical survey for renewal of certificate_

# Reason the survey has been carried out:

Completion of the new building

Change of flag

Conversion of the ship

Expiry of Safety and reason for

Partial Survey and reason for

Alterations, replacements or renewals affecting the Record of Approved Cargo Ship Safety Details_

Safety Equipment have been checked, and the corrections made have been listed overleaf_

Record of approved cargo ship safety details on board

Any recommendation issued or deficiencies noted completions of the survey are also listed overleaf

Yes

No

N/A

DOCUMENTATION:

Conditional certificate has been issued: Valid for	Months, Since	Until

The existing certificate has been extended until

Interim certificate valid for five months has been issued and valid until  	{expiry_date}

No certificate has been issued

This is to certify that the undersigned has examined the ship as reported herein, and has compared arrangements and equipment onboard with the Record of Approved Cargo Ship Safety Details and found to be in agreement therewith: _

# 1. SAFETY ITEMS (DOCUMENTATION, NAVIGATION, ETC. 1 AND 2)

1.	DOCUMENTATION

- Fire Control Plans posted on each side of the ship (FCP)

# Fire Control Plan (FCP) shows the following Information:

Fire Alarms	Detecting Systems	Fire Extinguishing Appliances  	  Means of Access to Different Compartments		Ventilating Systems

  - Is the FCP written in the national Language of the Crew and a translation into English in case of Crew’s language would not be English

  - Is the FCP up-to-date

  - Stability Data available

# Does the Trim and Stability Data include the Following:

      - Full Homogeneous Load “Departure” with 100% of all consumable* on board

- Full Homogeneous Load “Arrival” with 10% of all consumable* on board

- Ballast “ Departure” with 100% of all consumables* on board

- Ballast “ Arrival” with 10% of all consumables* on board

* Consumables:		Oil Fuel Tanks	Fresh Water Tanks	 Necessary Lube Oil Provisions for the Crew		Weigh and the Effects of the Crew

No

N/A

Yes

The loading conditions includes the following:

- Distribution of Liquid Cargo

- Resulting Draft

- Metacentric Height

- Stability Curves

- Minimum Stability Characteristic (Are they in accordance with the Article 10)

# Is the master provided with:

- Capacity Plan / Deadweight scale

- Cross Curves for different angles of heels

# PRACTICE, MUSTER AND DRILLS (ARTICLE 18)

- Date when last full crew muster for boat drills and fire drills took place (minimum once a month or 24 hours of leaving port whenever 25% of the crew have been replaced since the

last muster)_

- Log book entries are being made by the master

- The boat’s equipment, fire fighting, life saving appliances and other safety equipment were examined at that time and found complete and in satisfactory order working

- Were the conditions of all apparatus and systems and watertight integrity of the boats checked  as well as operation of the releasing devices

- The last occasion when the lifeboats were operated, indicate dates below

| LIFE BOAT NO. | TURNED OUT | LOWERED TO WATER | ENGINES OPERATED |
| --- | --- | --- | --- |

# FIRST AID (ARTICLE 19)

- Is the ship provided with a suitable first aid equipment

- Are there instructions in the form of a medical guide on board

- Is a stretcher included in the equipment (capable of enfolding the patient and being transferred from the lowest cargo hold to deck or from the ship to shore or boat

# NAVIGATIONAL EQUIPMENT (ARTICLE 23):

  - Is the Navigational Equipment according to record, and in good order

Yes	No	N/A

# IS THE SHIP FITTED WITH:

a)		 Are there Magnetic Compasses (for steering purpose and magnetic compass outside the bridge for use in taking bearings or azimuths, or alternatively, a single compass outside the bridge steering position with a reflector inside for steering purpose)

- Compasses adjusted (main and steering)	Last date of

- Echo sounder device

- Other means of obtaining depth (Hand lead properly marked and graduated up to 45 meters

(25 fathoms)

- Is the ship provided with appropriate charts and navigational publication suitable for the voyage It is to undertake

- Is the ship carrying an engine room, bridge and radio Log books

- Is the ship carrying the following publications:

  - International Regulation for Preventing Collision at Sea, 1972

  - International Code of Signals

  - Manual for Use by the Maritime Mobile and maritime Satellite Services

  - International Telecommunications Union (ITU) if it is fitted with a Radio Station

# NAVIGATION LIGHTS (COLREG 1972)

- Lights type approved or lights delivered with certificates (copy or certificates enclosed)_

- Lights are tested with power from main and emergency source

- If fitted, light failure indicator / alarm is tested

- Sidelights are screened to prevent light from being seen across the centerline

- Oil lights as listed in Record are in order

# SIGNALING LAMP (ARTICLE 21)

a)	Daylight Signaling lamp was tested on main and emergency source of power and was found Operational (For vessels more than 100 GRT)

Yes

No

N/A

LIFE – SAVING APPLIANCES (ARTICLE 17)

3.1	ROCKETS AND DISTRESS SIGNALS – (ARTICLE 17 (l))

a)		 Is the vessel provided with, at least, 6 parachute Distress Rocket Signals capable of producing a single bright red star at a high altitude and are they still valid (Usual validity, 3 year)_

Manufacture Date

Expiration Date

- Is the vessel provide with, at least, 6 Hands-Held Flares and are they still valid (Usual validity 3 years)

Manufacture Date

Expiration Date

- If the vessel is a Tanker or other ship carrying oil product shall carry, at least, 12 Parachute Distress Rockets (No Hand-Held Flares are allowed)

# LINE THROWING APPLIANCE (ARTICLE 17 (m)):

- Is the vessel provided with a Line Throwing Appliance with 2 lines and 2 projectiles capable of Throwing a line over a minimum distance of 230 m (70.12 Ft = 250 yards) (Just for ships operating  on Unrestricted Trade). (Usual validity 3 years)_

- The rockets of Line Throwing Appliance are still valid

Manufacture Date

Expiration Date

# LIFEBOAT/LIFERAFTS ARRANGEMENTS (ARTICLE 17 (a)):

-	According to the following table the, the ship is equipped with the option

| OPTION¹ |  | TRADING AREA | LB/CAPACITY | LR/CAPACITY | RB/CAPACITY | NOTES |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | A | Coastwise |  | 2/100% |  |  |
|  | B | Coastwise | 2/100% | 1/50% |  |  |
|  | C | Coastwise |  | 2/100% + 1/50% | 1 | The liferaft of 50% will not be required if the 2 liferaft of 100% are readily placed into the water on either side of the ship. |
| 2 | A | Unrestricted | 2/100% | 1/50% |  |  |
|  | B | Unrestricted |  | 2/100% + 1/50% | 1 | The liferaft of 50% will not be required if the 2 liferaft of 100% are readily placed into the water on either side of the ship |
|  | C | Unrestricted |  | 2/100% + 1/50% | 1 | 2 / 100% (Only for New Ships) (If the distance from the embarkation deck to the water in the lightest condition exceeds 8 m) |

  - LIFEBOAT AND LIFERAFT SPECIFICATION AND MAINTENANCE (ARTICLE 17 (b))

a)	Are the lifeboat’s sizes in accordance with the Article 17 (b) (1) (minimum Length = 4.9 m = 16 Ft)

- Are the Lifeboats, Liferafts of an approved type

- Is their Launching-gear of an approved design and construction complying with the applicable requirement of SOLAS according to the date of construction

Yes

No

N/A

- Are the skates or other suitable means provided to facilitate launching

- Was the equipment and provisions of the lifeboats inspected

Provision’s Manufacture Date	Expiration Date

- Was the equipment and provisions of the liferafts inspected

Provision’s Manufacture Date	Expiration Date

# LIFEBOATS / RESCUE BOATS – (ARTICLE 17 (c))

|  | No. 1 | No 2 |
| --- | --- | --- |
| a) Lifeboats now turned out and lowered to embarkation deck |  |  |
| b) Lifeboats now turned out and lowered to water |  |  |
| c) Hull inside and outside, and lifting hooks inspected |  |  |
| d) When lowered to water, the Des-engagement gear readily released the lifeboat |  |  |
| e) Portable or built-in aircases removed found or placed in good conditions, replaced and Secured. |  |  |
| f) Air Cylinders in covered boats fully charged (if provided) |  |  |
| g) Lifeboats marked with dimensions, name, port or registry and number of persons |  |  |
| h) Engines in motorboats operated |  |  |

  - LIFEBOAT EQUIPMENT (ARTICLE 17 (c)):

- Food rations (2 years), tinned water (3 years) and pyrotechnics (3 years) are valid

- Fire extinguisher in motorboat is serviced

- Motor run ahead and reverse

- Fuel tank pipes are checked

- Sufficient fuel for 4 hours operation provided

- All equipment is in order and secured

# AVAILABILITY OF LIFEBOAT AND LIFERAFTS (ARTICLE 17 (f)):

a)	All items of lifesaving equipment are readily available for use in an emergency

- Are the Liferafts stowed in float-free position

- Are the liferafts fitted with Hydrostatic Release Units

_

Detail Instructions for operating liferafts are conspicuously posted

- Detail Instructions for operating Lifeboats are conspicuously posted

# MARKING OF LIFEBOATS AND RESCUE BOATS (ARTICLE 17 (g)):

a)	Are Lifeboats or Rescue Boats properly marked (on both bows) with the name of the parent ship, port of registry, principal dimensions and Carrying Capacity

Yes

No	N/A

# DAVITS FOR LIFEBOATS AND LIFERAFTS (ARTICLE 17 (h)):

- All shaves, blocks, falls and all moving parts are free and well lubricated

- Hidden parts of wire falls have been inspected

- Date or reversal or renewal of falls

# EMBARKATION INTO AND ILLUMINATION OF LIFEBOATS/ LIFERAFTS (ARTICLE 17 (i)):

- Does the vessels has a rope ladder for each set of davits capable of reaching light water line

- Suitable lights for illuminating Liferafts and/ or Lifeboats are provided tested with the emergency source of power and found in good order

# LAUNCHING AND EMBARKATION ARRANGEMENTS (ARTICLE 17 (h), (i)):

- All blocks, chains and shackles checked

- Boat lashings and release arrangements in order

- Lifelines fitted to davit spans checked

- Embarkation ladders checked

- Lighting at boat / raft stations and oversides tested on mains emergency source or power

- Alarm system tested

- Means of preventing water discharge into boats verified

# LIFERAFTS (ARTICLE 17 (a)):

Servicing of inflatable (due every 12 months) (Hydrostatic release to be serviced together with raft)

| No. | Maker’s name and Serial No. | No. of Persons | Date of Service (Rafts) | Date of Next Inspection (Rafts) | Rafts’ Hydrostatic Release Expiration Date |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |

Yes

No

N/A

OTHER LIFE – SAVING APPLIANCES – (ARTICLE 17 (j), (k))

- Lifejackets examined and found in order

- Lifejackets readily accessible and their position plainly indicated

- All Lifejackets are fitted with a non-Corrosive whistle, attached firmly to a cord, and lights

- Lifebuoys in good condition, and marked with name and port of registry

- Lights and lines in order, and smoke signal valid

- Are Lifebuoys and Lifejackets of approved type

- Quick release from bridge in order

# 4.0. FIRE PUMPS (ARTICLE 14C):

- Is the vessel provided with a main Fire Pump

- Is the main Fire Pump a power driven pump

- Is the main Fire Pump one of the following pumps_

Bilge Pump,	General Service Pump,	Ballast pump,		Sanitary Pump Other (which)_

- Is the main fire pump delivering a minimum pressure of 2.05 Kg/Cm² = 2.01 Bar = 201Kpa = 29.15 Psi_

- Is the main Fire pump capable of delivering a 12m = 39.4Ft of jet of water

- Is the vessel provide with an Emergency Fire Pump (only for vessel undertaking unrestricted trade)

- Is the Emergency Fire Pump a power driven pump operated by a means independent from the vessel’s Main source of power or a hand operated pump

- Is the Emergency fire Pump located outside the machinery space

- Is the Emergency Fire pump capable of delivering a 6m = 19.7.Ft of jet of water

# FIRE MAIN, HYDRANTS, HOSES AND NOZZLES (ARTICLE 14 (d)):

- Is the ship fitted with the minimum number of hoses, hydrants, and nozzles

- Are hoses manufactured with a suitable material

- Are Hydrants located in accordance with the Regulations

Yes	No	N/A

- Are hoses kept in a conspicuous position near the Hydrants, which they are intended to be used

- Are Hydrants and hose stowage boxes suitable labeled and painted red

# FIXED FIGTH FIGHTING SYSTEM (ARTICLE 14 (e)):

a)	Is there, in any Oil-Fired boiler room and in any unattended propulsion machinery space a suitable Fixed Fire Fighting System installed

b)    An Exemption of Fixed Fire Fighting System has been granted by the Administration (Just for vessels   engaged in Coastwise or Coastal Trade)

# FIRE EXTINGUISHERS (ARTICLE 14 (f)):

- Is the vessel provided with a sufficient number of approved Portable Fire Extinguishers for being used

in accommodation and service spaces with at least one provided at each deck level

- In each space containing Internal Combustion type machinery is there, as a minimum, 2 Fire Extinguishers

- Are generally, all Fire Extinguishers specifically intended for being used in a particular space, stowed near the entrance/exits to said space

- All extinguishers has been pressure tested and recharged once a year

- All Fire Fighting equipment has been clearly labeled for its specific purpose either in the predominant language of ship’s personnel and in English or by means of Self-explanatory diagrams

- Is the Fireman’s Outfit complete

- Was the Self – containing Breathing apparatus tested

- Air cylinders are fully charged

- Smoke helmet or smoke mask Functioning with air jump and hose tested

# MACHINERY AND ELECTRICAL INSTALLATIONS (ARTICLE 11)

- Are the indicator fitted in the wheelhouse

- Are there means of communication provided between ship’s bridge and the Engine Room

- Is the ship provided with an efficient plant capable of pumping from and draining any watertight Compartment

- Is the ship fitted with two independent power operated bilge pumps

- Were bilge pumps tested and found in good order

- Pipes and valves were tested and found in adequate condition

Yes

No	N/A

- Automatic remote bilge high level alarm (unattended E/R) if provided, tested and found in good order

- Is the ship fitted with a source of electrical power, a distribution switchboard and a system of electric Wiring suitably protected to provide power to the machinery, heating, lighting, ventilation, alarms and Other circuits required onboard

- All main generators was tested and found operational

- Was the emergency source of power tested and found operational

- Emergency source of power was capable of providing light for:

Alley-ways	Stair-Ways	Main machinery space

Navigating Bridge		Chart Room		Liferafts, Lifeboats, Rescueboat stowage position Steering Gear Space	Navigation Lights	Alarm Signal Circuits, specially the general Alarm Main Generating Space

- Emergency lights are adequate and found operational

- Visible condition of electric machinery and wiring were found adequate to minimize the risk of electric Shock or short circuit

# ANCHOR AND MOORING EQUIPMENT (ARTICLE 12):

- Windlass and Stoppers (if provided) were tested and found operational

- Capstans and mooring winche (if provided) tested and found operational

# STEERING GEAR (ARTICLE 13):

- Is the ship fitted with a main steering gear capable of guiding the ship at maximum speed

- Was the rudder angle indicator found in good order and fitted in the Bridge

- Was the Main Steering Gear tested and found operational

- Was the Emergency Steering Arrangement tested and found in good order

- Are the Instructions for emergency Steering Gear Arrangement available and adequate and shows how

the system brought into effect

# REMOTE CONTROL AND ALARMS (ARTICLE 15):

- Is the ship provided with means for stopping ventilating fans serving machinery and cargo spaces

- Is the ship provided with a means for closing all doorways, ventilators, annular spaces around the funnel

- and other openings to such spaces

- Are those stopping and closing devices capable of being operated from outside of such spaces in case of Fire

Yes	No	N/A

- Forced and Induced Draft fans, Oil Pumps, Purifiers and other Oil handling equipment are in capacity of being operated with a remote control situated outside the spaces concerned

- Is the ship used for unattended propulsion machinery

- Is the ship fitted with an automatic remote fire alarm (In case the ship is used as unattended)

# PILOT LADDER (ARTICLE 22):

a)		 Is the vessel carrying an efficient pilot ladder for the purpose of enabling pilots to embark and disembark safely

# ACCOMODATION REGULATION (ARTICLE 25):

a)		 Is the vessel in conformity with the requirements of the International Labour Organization (ILO) Convention concerning Crew Accommodation on board ships ratified by the administration

# PROPULSION MACHINERY (ARTICLE 11):

- Was main engine tested and found in good order

- Means of going astern tested and found in order

- Speed controls and gauges found in order, remote RPM and direction or pitch indicator in the wheelhouse synchronized with engine

- Was the Propulsion auxiliaries (pump, coolers, etc.) found in good order

- Piping systems found in order, without leaks, with valves operational and filters cleaned

# 11.1. BOILERS AND PRESSURE VESSELS

## Applicable to boilers with a power greater than 175 kW (T/h) or pressure vessels of a Capacity above   500 liters (about 130 gallons)

- Steam Piping found in order, without leaks, valves operational, auxiliary tanks in order, drains and Strainers in good order too

- Flammable or toxic gas containers located outside the accommodation or the main machinery spaces

# HULL AND DECK EQUIPMENT

12.1 GENERAL

- If the vessel is classed, indicate Class Society and date of expiry of valid Class Certificates

- Examinations under sections 12.3, 12.4, 12.5 and 12.6 may be omitted in vessels classed by one of the Class Societies which area authorized by the Republic de Panama to issue technical certificates on its behalf_

Yes	No	N/A

Examinations under section 12.4 may be omitted if the vessel is holding a valid Load Line Certificate Issued on behalf of the Government whose flag the ship is entitled to fly

- Indicate date and place of last dry-docking

# ANNUAL HULL SURVEY

## This survey is to be carried out for issuing initial or for the endorsement of the certificate

a)	Overside over the waterline parts examined all around the vessels and found to be in an overall good condition

# b)	The following items were especially examined and found in the following condition:

- Degree of visible corrosion

- Plates around chafing areas

- Fenders, if fitted

- No apparent dents at bulkheads locations or distortions in way of major internal structural members

- Weather decks examined from above and found in good condition

- Watertight bulkheads examined from both sides and found in good condition (if accessible)_

- Visual condition of ladder ways and railing

# DRY – DOCK SURVEY (SEE 12.1.B)

## This survey is to be carried out for the issue of the certificate, or as required by the Regulation.

- Underwater parts of hull examined and found in the following condition

- The Following items were especially examined and found in the following conditions:

1.	States degree of visible corrosion

- Rudder examined and found in good condition, rudder pintle clearances corrected of found in excess of

10 % of pintle diameter or 10mm, whichever is lesser_

- Propeller examined and found in apparent good condition

- Any cemented areas to be tested with hammer for proper adherence

- The internal sides of the boundaries of all holds and ballast, fuel- oil and cargo tanks examines and found In good condition. All oil tanks must be gas free and clean prior to the examination

Yes

No

N/A

Insulation panels and / or wooden deck coverings, if provided, to be removed at locations near the peak Bulkheads and near the center of the vessel and the underlying plates are found in good condition. This operation need not be carried out in vessels less than eight (8) years old.

# The following locations were examined:

Double Bottom

Upper side of decks

Insulated reefer cargo spaces

- Main structural members were gauged, (These measurements shall only be carried out wherever heavy corrosion wastage is apparent. Where the wastage is found to exceed 25 % of the original thickness, the structure should be renewed): The Following locations were tested:

  - Wastage	% 2.	Wastage	%

# LOAD LINES (SEE 12.1. c)

- All doors, sidelights and windows in the first two superstructure levels above the uppermost continuos deck examined and found in good condition

- Main hull sidelights and shell openings examined and found in good condition

- Ventilation inlets and outlets examined and found in good condition

- Tank vents examined and found in good condition

- Hatch covers examined and found in good condition

# CARGO GEAR (SEE 12.1. b)

- Masts, booms, cranes and supporting structures examined and found in good condition

- Blocks and chocks examined and found in good condition

- Wires of standing and running rigging examined and found in good condition

- Winches operated and found in good condition

# ANCHORS, CABLES, ETC

- Visible condition of anchors and chains or wires examined and found adequate

- Visible condition of mooring lines found adequate_

## PARTICULARS OF ANY SPECIAL FEATURES OR ADDITIONAL INFORMATION APPERTAINING TO SAFETY EQUIPMENT, OR CONSTRUCTION AND OR OUTSTANDING RECOMMENDATIONS AND DEFICIENCIES:

- ALTERATIONS., REPLACEMENTS OR RENEWALS IN THE RECORD OF APPROVED CARGO

SHIP SAFETY DETAILS:

| Item No. According to the Record | The Record on board is corrected as follows |
| --- | --- |

- PARTICULARS OF OUTSTANDING SAFETY ITEMS REQUIRING ATTENTION OF SURVEY:

| Item No. According to the Record | Remarks: |
| --- | --- |

ALL PARTS SURVEYED WERE FOUND TO BE IN AN EFFICIENT AND GOOD CONDITION AND THE UNDERSIGNED RECOMMENDS:

YES

NO

- The issue of a full term certificate

- The issue a full certificate after the recommendation have been carried out

- No full term certificate to be issued before the survey has been completed (see list of outstanding items, and deficiencies above)

Place and Date of Survey	   GR CLASS Surveyor Name – ID No.