# GR CLASS CLASSIFIED FOR STANDARD (GR CLASS)

REPORT OF CARGO SHIP SAFETY COSNTRUCTION SURVEYS ANNEX III - OIL TANKERS

| SHIP NAME: | IMO No.: | CALL SIGN: | FLAG: | CLASS No. |
| --- | --- | --- | --- | --- |

III. OIL TANKERS

1. ANNUAL SURVEYS

a. GENERAL (EITHER FOR SINGLE OR DOUBLE HULL) (A. 744 (18), Annex B, Part A & B)

- Please, complete section 1(A, B & C) for general cargo

- Additionally please, verify the following:

  - Examining the towing arrangements 20.000 DW and over (SOLAS 74/00 reg.II-1/3-4);

  - Confirming that the corrosion prevention system fitted to dedicated ballast water tanks is maintained (SOLAS 74/00 reg.II-1/3-3);

- Confirming, when appropriate, that the requisite arrangements to regain steering capability in the event of the prescribed single failure are being maintained (SOLAS 74/88 reg.II-1/29);

- Examining , as far as practicable, the cargo, bilge ballast and stripping pump for undue gland seal leakage, verification of proper operation of electrical and mechanical remote operating and shutdown devices and operation of cargo pump room bilge system, and checking that pump foundations are intact;

- Confirming that the pump room ventilation system is operational, ducting intact, dampers are operational and screens clean;

- Verifying that installed pressure gauges on cargo discharge lines and level indicator systems are operational;

- Examining access to bow arrangement (SOLAS 74/00 reg.II-1/3-3)

- confirming, when appropriate, that a complete file of the enhanced survey reports and the Condition Evaluation Report are on board;

- Confirming, when appropriate, and as far as is practicable when examining internal spaces that the means of access to cargo and other spaces remain in good condition. (SOLAS 74/00/02 reg.II-1/3-6);

.1 Examination of the hull

  - Confirm that the examination of the hull plating and its closing appliances were carried out as far as can be seen;

  - Confirm that an examination of watertight penetrations was carried out as far as practicable;

.2 Examination of weather decks

  - Confirm that an examination of cargo tank openings including gaskets, covers, coamings and flame screens have been made;

  - Confirm that an examination of cargo tank pressure/vacuum valves and screens have been made;

  - Confirm that an examination of flame screens on vents to all bunker and oily slop tank was made;

  - Confirm that an examination of cargo, crude oil washing, bunker and vent piping system, including vent mast and headers have been made;

.3 Examination of Cargo pump-room and pipe tunnel

  - Confirm that an examination of all bulkheads for signs of oil leakage, fractures and, in particular, the sealing arrangements of all penetrations of bulkheads;

  - Confirm that an examination of the condition of all piping systems and pipe tunnels have been made;

.4 Examination of ballast tanks1

  - Tankers exceeding 15 years of age:

    - Confirm that an internal examination of all ballast tanks adjacent to (i.e. with a common plane boundary) a

CARGO or FUEL TANK with any means of heating; (see foot note 1). Please describe spaces examined

|  | B.T. examined at this time |
| --- | --- |

|  | B.T. examined at this time |
| --- | --- |

|  | B.T. examined at this time |
| --- | --- |

    - Confirm that an internal examination of all ballast tanks which were found, at the previous intermediate or Renewal survey, to have no substantial corrosion within the tank and which were found in compliance with either the following conditions:

      - Coating in good conditions; Describe result of the examination of the below table. Use SS-001/SS-002 to give additional information if necessary;

|  | B.T. examined at this time |
| --- | --- |

|  | B.T. examined at this time |
| --- | --- |

|  | B.T. examined at this time |
| --- | --- |

      - Coating of common boundary, including adjacent structures, in GOOD condition and the coating of the

remaining parts of the tank if in FAIR conditions. Use SS-001/SS-002 to give additional info. if necessary;

|  | B.T. examined at this time |
| --- | --- |

|  | B.T. examined at this time |
| --- | --- |

|  | B.T. examined at this time |
| --- | --- |

- For the hull, machinery and equipment the completion of the annual survey should consist of:

  - After the satisfactory survey, the Cargo Ship Safety Construction Certificate was endorsed;

  - If the survey shows that the condition of a ship or its equipment is unsatisfactory, use SS-001/SS-002 to inform HO.

1 Examination of ballast tanks should be carried out when required as a consequence of the result of the renewal and intermediate survey. When extensive corrosion is found, thickness measurements should be carried out.

When substantial corrosion is found, the extent of thickness measurement should be increased in accordance with the requirement of Annex 4 of A. 744(18) as amended.

2. INTERMEDIATE SURVEYS

a. GENERAL (EITHER FOR SINGLE OR DOUBLE HULL) (A. 744 (18), Annex B, Part A & B)

- Please, complete section 1(A, B & C) for general cargo

- Additionally please, verify the following:

.2.1 Testing the insulation resistance of electrical circuits in dangerous zones such as cargo pump rooms and areas adjacent to cargo tanks, but in cases where a proper record of testing is maintained, consideration should be given to accepting recent readings.

- For weather decks, an examination as far as practicable of cargo, crude oil washing, bunker, ballast, steam and vent piping systems as well as vent mast and headers. Should there be any doubt as to its condition when examining the various piping systems, the piping may be required to be pressure tested, gauged or both. particular attention is to be paid to repairs such as welded doublers;

.3.1 Oil tankers of 5 to 10 years of age

    - For tanks used for salt water ballast, and overall survey2 of representative tanks3 selected by the   surveyor should be carried out. If the overall survey of salt water ballast tanks reveals no visible structural defects, the examination may be limited to verification that protective coatings remain effective; Please use the following table to inform which spaces were examined and use SS-001/SS-002 to give additional details of extent/condition

|  | B.T. examined at this time |
| --- | --- |

|  | B.T. examined at this time |
| --- | --- |

|  | B.T. examined at this time |
| --- | --- |

    - Where POOR coating condition, corrosion or other defects are found in salt water ballast tanks or

where a protective coating was not applied from the time of construction, the examination should be extended to other ballast tanks of the same type. Please use the following table to inform which spaces were examined and use SS-001/SS-002 to give additional details.

|  | B.T. examined at this time |
| --- | --- |

|  | B.T. examined at this time |
| --- | --- |

|  | B.T. examined at this time |
| --- | --- |

    - In salt water ballast tanks where a protective coating is found POOR condition and it is not renewed, where soft coating has been applied, or where a protective coating was not applied from the time of construction4, the tanks in question should be examined and thickness measurement carried out as considered necessary at annual Intervals. Use SS-001/SS-002 to indicate the spaces found POOR and where UTM has

been taken (if any);

3.2 Oil tankers of 10 to 15 years of age

    - Please, complete section (2.a.3.1 above) of this survey report

2 Overall survey is a survey intended to report on the overall conditions of the hull structure and determine the extent of additional close-up surveys.

3 Representative tanks are those which are expected to reflect the condition of other tanks of similar type and service and with similar corrosion protection system. When selecting representative spaces account should be taken of the service and repair history on board and identifiable critical and/or suspect areas.

4 Delete as necessary.

    - Additionally please, verify the following:

.1 An overall survey of at least two Representative Cargo Tanks. Use SS-001/SS-002 to give details;

|  | B.T. examined at this time |
| --- | --- |

|  | B.T. examined at this time |
| --- | --- |

|  | B.T. examined at this time |
| --- | --- |

2. For Ballast Tanks, where fitted and Overall Survey of all such tanks is to be carried out. If such survey reveals no visible structural defects, the survey may be limited to a verification that the protective coating remain efficient. Use SS-001/SS-002 to give details;

|  | B.T. examined at this time |
| --- | --- |

|  | B.T. examined at this time |
| --- | --- |

|  | B.T. examined at this time |
| --- | --- |

b. SINGLE HULL TANKERS (This section is to be completed (as applicable) in addition of section 2.a.)

    - Extent of close-up survey

      - Ballast tanks: the same extent as previous renewal survey. Use SS-001/SS-002 to give details which spaces were examined according to the below items and inform the condition of them;

        - All web frames in all ballast tanks;

# All transverse bulkheads in all ballast tanks;

      - Cargo tanks: Two combined cargo/ballast tanks. The extent of the survey should be based on the record of the previous renewal survey and repair history of the tank. Use SS-001/SS-002 to give details of which spaces were examined according to the below items and inform the condition of them;

|  | C.T. examined at this time |
| --- | --- |

|  | C.T. examined at this time |
| --- | --- |

|  | C.T. examined at this time |
| --- | --- |

      - Thickness Measurement (UTM): See MTO section of vessel’s CSSR. Use SS-001/SS-002 to give details

.i The minimum requirements for UTM at the intermediate survey are areas found to be suspect areas5 at the previous renewal survey. Where substantial corrosion6 is found, the extent of UTM should be increased in accordance with the requirement of A. 744 (18), Annex B, Part B, Annex 4;

.3.3 Oil tankers exceeding 15 years of age7

3.3.1 Please, complete section 3.a of this survey report;

5 Suspect areas are locations showing substantial corrosion and/or are considered by the surveyor to be prone to rapid wastage.

6 Substantial corrosion is an extent of corrosion such that the assessment of corrosion pattern indicates a wastage in excess of 75% of allowable margins, but within acceptable limits.

7 The requirement of the intermediate survey should be to the same extent as the previous renewal survey as required in section 3 of this survey report. However, testing of cargo ans ballast tanks is not required unless deemed necessary by the attending surveyor.

c. DOUBLE HULL TANKERS (This section is to be completed (as applicable) in addition of section 2.a.)

    - Extent of close-up survey

      - Ballast tanks: the same extent as previous renewal survey. Use SS-001/SS-002 to give details which spaces were examined according to the below items and inform the condition of them;

        - All web frames in all ballast tanks;

# All transverse bulkheads in all ballast tanks;

      - Cargo tanks: the same extent as previous renewal survey. Use SS-001/SS-002 to give details which spaces were examined according to the below items and inform the condition of them;

.i Close-up survey in two cargo tanks8.

      - Thickness Measurement (UTM): See MTO section of vessel’s CSSR. Use SS-001/SS-002 to give details

.i The minimum requirements for UTM at the intermediate survey are areas found to be suspect areas9

at the previous renewal survey. Where substantial corrosion10 is found, the extent of UTM according to

A. 744 (18) Annex B, Part A, Annex 5 of should be increased in accordance with the requirement of annex 4;

.3 Oil tankers exceeding 15 years of age11

.3.1 Please, complete section 3a of this survey report;

3. RENEWAL SURVEYS12

a. GENERAL (EITHER FOR SINGLE OR DOUBLE HULL)

- Please, complete section 1(A, B & C) for general cargo;

- Additionally please, verify the following:

  - Crude Oil Washing system (COW) piping was examined and operationally tested to working pressure to the attending surveyor’s satisfaction to ensure that tightness and condition remain satisfactorily;

  - Cargo piping on deck was examined and operationally tested to working pressure to the attending surveyor’s satisfaction to ensure that tightness and condition remain satisfactorily;

  - Cargo piping within the above tanks and spaces were examined and operationally tested to working pressure to the attending surveyor’s satisfaction to ensure that tightness and condition remain satisfactorily*;

  - Ballast piping within the above tanks and spaces were examined and operationally tested to working pressure to the attending surveyor’s satisfaction to ensure that tightness and condition remain satisfactorily*;

  - All cargo tanks, ballast tanks, pump-rooms, pipe tunnels, cofferdams and void spaces bounding cargo tanks,

decks and outer hull should be examined. This examination may be supplemented by UTM as necessary;

- After a satisfactory survey, the Cargo Ship Safety Construction Certificate should be issued.

- if the survey shows that the condition of a ship or its equipment is unsatisfactory, use SS-001/SS-002 to inform HO.

.5 Extent of overall survey

.5.1 An overall survey of all integral tanks and spaces should be carried out. Please, use SS-001/SS-002 to give details of which spaces were examined and their conditions.

8 The extent of close-up survey should be based on the records of the previous renewal survey and repair history of the tanks. The surveyor may extend the scope of close-up survey as deemed necessary taking into account the maintenance of the tanks under survey, condition of the corrosion prevention system and also in particular, tanks having structural arrangement or details which have suffered defect in similar tanks or on similar ships according to the available information, and in tanks which have structures with reduced scantling in association with a corrosion prevention system approved by the Administration.

9 Suspect areas are locations showing substantial corrosion and/or are considered by the surveyor to be prone to rapid wastage.

10 Substantial corrosion is an extent of corrosion such that the assessment of corrosion pattern indicates a wastage in excess of 75% of allowable margins, but within acceptable limits.

11 The requirement of the intermediate survey should be to the same extent as the previous renewal survey as required in section 3 of this survey report. However, testing of cargo ans ballast tanks is not required unless deemed necessary by the attending surveyor.

12 The renewal survey may be commenced at the fourth annual survey and be progressed during the succeeding year with a view to completion by the fifth anniversary date. As part of the preparation of the renewal survey, the survey program should be dealt with in advance of the renewal survey. The thickness measurement should not be held before the fourth annual survey.

* Special attention should be given to any ballast piping in cargo tanks and cargo piping in ballast tanks and void spaces and surveyor should be advised on all occasion where this piping, including valves and fittings are to be open during repair periods and can be examined internally.

.6 Extent of Close-up survey

.6.1 Verify that close-up survey, according to A. 744 (18), Annex B, Part A, Annex 1 as amended has been carried out. Please, use SS-001/SS-002 to give details which spaces were examined and their conditions.

.7 Extent Thickness Measurement

  - Verify that thickness measurement, according to A. 744 (18), Annex B, Part A, Annexes 2 & 4 as amended, has been carried out. Please, use SS-001/SS-002 to give details which spaces were measured

  - Verify that areas with substantial corrosion (if any), has been identified and proper entry has been made using SS-001/SS-002 as applicable.

.8 Extent of tank pressure testing

.8.1 Verify that pressure tank testing13, according to A. 744 (18), Annex B, Part A, Annex 3 as amended, has been carried out. Use SS-001/SS-002 to give details which spaces were tested

.9 Please, additionally follow the guidelines (GUI-ESP-OT)

4. DRY-DOCK SURVEYS14

.1 Please, complete section 4 of form CCC-001 for general cargo

5. LONGITUDINAL STRENGHT EVALUATION15

| Transverse Section | Frame No. | Deck/Bottom | Measured Area (cm²) | As built Area (cm²) | Lost of Area (%) | Remarks* |
| --- | --- | --- | --- | --- | --- | --- |

* Note: In case the transverse section thickness measurement (UTM) report shows that loss of area is less than 10% enter in “remark”

column: “Areas not calculated as compliance checked from UTM report”.

If loss area is more than 10%, section moduli must be grater than required section moduli.

| Transverse Section | Frame No. | Deck/Bottom | Measured Modulus (cm³) | Required Modulus (cm³) | Remarks |
| --- | --- | --- | --- | --- | --- |

Place and Date of Survey	GR CLASS Surveyor Name – ID No.

13 The surveyor may extend the tank pressure testing as deemed necessary. Generally, the pressure should correspond to a water level to the top of access of hatches for Cargo tanks, or top of air pipes for ballast tanks.

14   Asurveyindry-dockshouldbeapartoftherenewalsurvey.Thereshouldbeaminimumoftwoinspectionsoftheoutsideoftheship'sbottomduringthe5-yearperiodofthecertificate.Inallcases,themaximumintervalbetweenbottominspectionsshouldnotexceed36months.Forships

of15yearsofageandover,inspectionoftheoutsideoftheship'sbottomshouldbecarriedoutwiththeshipindrydock.Forshipsoflessthan15yearsofage,alternateinspectionsoftheship'sbottomnotconductedinconjunctionwiththerenewalsurveyduringtheperiodicalsurveymaybe carriedoutwiththeshipafloat.Inspectionoftheshipafloatshouldonlybecarriedoutwhentheconditionsaresatisfactoryandtheproperequipmentandsuitablyqualifiedstaffsareavailable.

If a survey in dry-dock is not completed in conjunction with the renewal survey during periodical survey or if the 36 month maximum interval referred to in is not complied with, the Cargo Ship Safety Construction Certificate should cease to be valid until a survey in dry-dock is completed.{expiry_date}

15 The evaluation of longitudinal strength is applicable to oil tankers of 130 m in Length and upwards and over 10 years of age. The longitudinal strength of the ship’s hull girder should be evaluated in

compliance with the requirements of A. 744(18), Annex B, Part A / B, Annex 12 on the basis of thickness measured, renewed or reinforced, as appropriate during the renewal survey of Safety Construction. The condition of the hull girder for longitudinal strength evaluation should be determined in accordance with the methods specified in the Appendix 3.