# GR CLASS CLASSIFIED FOR STANDARD (GR CLASS)

SURVEY REPORT ON SOLAS SURVEYS – VDR & S-VDR RADIO TECHNICIAN SURVEY

.

(ORIGINAL TO BE RETAINED ON BOARD UNTIL NEXT CEC SURVEY)

| Name of Ship: | Port of Registry: | GT: | Call Sign: | Date Keel Laid: |
| --- | --- | --- | --- | --- |
| Official No: | IMO No.: | INMARSAT ID No.: |  |  |

| VDR/S-VDR Note- Check Y for success, N for failure or N/A for Non fitted interfaces in these boxes, as appropriate. |  |
| --- | --- |
| Voyage Data Recorder Details |  |
| Manufacturer: |  |
| Model: |  |
| System Serial number: |  |
| Software version Number: |  |
| Date fitted: |  |

| Inspection Details |  |
| --- | --- |
| Name of Radio Technician: |  |
| Company: |  |
| Inspection Date: |  |
| Inspection Location: |  |

| 1. Pre- Existing Alarms: | Y | N | N/A |
| --- | --- | --- | --- |
| a. Confirm that no alarms were present at start of procedures |  |  |  |

| 2. Power supply alarm check: | Y | N | N/A |
| --- | --- | --- | --- |
| a. Remove sources of external power. Confirm that alarm is activated. |  |  |  |
| b. Record Time (hh.mm) |  |  |  |

| 3. Reserve power source check: | Y | N | N/A |
| --- | --- | --- | --- |
| a. Allow VDR to continue running for 1 hour 55minutes from”2” above. Confirm that equipment is still operating at this time, with no additional alarms. |  |  |  |
| b. Record Time (hh.mm) |  |  |  |

| 4. Reserve power source shutdown check: | Y | N | N/A |
| --- | --- | --- | --- |
| a. 2 hours 05 minutes from “2” above confirm that the VDR has automatically stopped recording. |  |  |  |
| b. Record Time (hh.mm) |  |  |  |

| 5. Battery Expiry Dates: Expiry dates (where applicable) |  | Y | N | N/A |
| --- | --- | --- | --- | --- |
| Acoustic Beacon: |  |  |  |  |
| Reserve power source: |  |  |  |  |

| 6. Acoustic beacon test: | Y | N | N/A |
| --- | --- | --- | --- |
| a. Using manufacturer’s test equipment confirm that acoustic beacon is functional or by the substitution of a certified fully operational unit. |  |  |  |

| 7. Overall Conditions of equipment: |  | Y | N | N/A |
| --- | --- | --- | --- | --- |
| Sub unit | Notes on Conditions |  |  |  |
| Protective Capsule |  |  |  |  |
| External Cables |  |  |  |  |
| Main Unit |  |  |  |  |

| 8. Interfaces: Operation and recording: |  | Y | N | N/A |
| --- | --- | --- | --- | --- |
| Date and Time | Preferably external to ship (e.g. Global Navigation Satellite System.) |  |  |  |
| Ship’s position | Electronic Positioning System |  |  |  |
| Speed (Through or over ground1) | Ship’s designated speed and distance measuring equipment |  |  |  |
| Heading | Ship’s compass |  |  |  |
| Bridge audio | 1 or more bridge microphones |  |  |  |
| Communication Audio | VHF |  |  |  |
| Radar Data-post display selection | Master radar display |  |  |  |
| Water depth | Echo sounder |  |  |  |
| Main Alarms2 | All mandatory alarms on bridge |  |  |  |
| Rudder order and response | Steering gear and autopilot |  |  |  |
| Engine order and response | Telegraphs, controls and thrusters |  |  |  |
| Hull openings Status | All mandatory status information displayed on bridge |  |  |  |
| Watertight and fire door status | All mandatory status information displayed on bridge |  |  |  |
| Acceleration and hull stresses | Hull stresses and response monitoring equipment where fitted |  |  |  |
| Wind speed and direction | Anemometer where fitted |  |  |  |

| 9. Change or repair of sensors: | Y | N | N/A |
| --- | --- | --- | --- |
| Check maintenance records of VDR |  |  |  |
| Confirm any defects property rectified |  |  |  |
| Person Authorized by the Manufacturer: Date: |  |  |  |
| Ship´s Representative: Date: |  |  |  |
| If the manufacturer does not complete a review and issue a completed test report within 45 days, this test report should go forward for certification. |  |  |  |

| 10. Manufacturer’s analysis: | Y | N | N/A |
| --- | --- | --- | --- |
| Note- This confirms the endorsement by the manufacturer of the test and that the master record/database has been checked. |  |  |  |
| Manufacturer’s Analysis of 12 hours log is attached and in accordance with International Electro technical Commission (IEC) 61996 Maritime navigation and radio communication equipment and systems – Shipborne voyage data recorder (VDR)- Performance requirements – Methods of testing and required test results section 4.6 – Data items to be recorded (resolution A.861(20).Section 5.4). Confirmation that all data is available throughout the 12-hour recording. |  |  |  |

1 Either speed through the water or speed over the ground needs to be recorded from the ship´s speed and distance measuring equipment, along with an indication of which one it is.

2 For VDR´s (Not S-VDR) Verification that recorded Main Alarms of unmanned Engine room vessels includes the bridge alarms per MSC Resolution A. 830(19) and MSC Resolution A. 686(17) – Table 9.1.1.

Date and Time of the above Log:

| 11. Observations and additional manufacture’s requirements: | Y | N | N/A |
| --- | --- | --- | --- |
| Note- This specifically provides for the logging of significant events that may have occurred on board since the previous test, including the refitting of equipment or major unit change to e4xisting equipment- Any or all of which may have an impact on the availability or quality of the VDR/S-VDR input signal. |  |  |  |

This performance test was conducted in accordance with SOLAS regulation V/18.8 and forms part of the procedure for the issue of the Annual Performance Test Certificate. The results, information and any comments should be relayed to the manufacturer in accordance with the instructions contained within the Operation Manual. Subject to satisfactory results, an Annual Performance Test Certificate will then be issued.

{expiry_date}

This report may be considered as the certificate of compliance issued by the testing facility, as required by SOLAS V/18.8, provided it is properly sign and dated subject to the equipment being maintained in appropriate operational condition. In accordance with the harmonization of certificates this certificate will remain valid until next annual re-validation inspection when a new checklist is to be completed and left on board.{expiry_date}

Radio Technician Remarks:

# Radio Technician’s Printed Name	Radio Technician’s Signature

Radio Firm Name	Place of survey and Date