GR CLASS CLASSIFIED FOR STANDARD (GR CLASS) PROCESS INSTRUCTION CHECK SHEET ON

ANNUAL SURVEY FOR MODU CODE

Initial Survey

Periodical Survey

(Annual)	Renewal Survey

Additional

Survey (Occasional)

| Name Of Ship | Port Of Registry |  | Gross Tonnage | Year Keel Laid |
| --- | --- | --- | --- | --- |
| DISTINCTIVE NUMBER OR LETTERS |  | Patente Number And Date: |  |  |

Details Of Persons Requesting Survey And To Whom Certificates Are To Be Sent

OWNERS/MASTER

AGENTS/OPERATORS

--

Date of Survey   /   commenced:    		 completed:	 			        Name/Number  of Surveyor:	 		 Place of Survey:

Note:	Mark in box	Fit or Yes

Unfit or No	Not Applicable

# GENERAL

  - Surveys and Certification

    - Have you confirmed that no significant change has been made in the structure fittings, arrangements, safety equipment, and other equipment without approval, except the direct replacement of such equipment and fittings for the purpose of repair and maintenance?

    - Have you confirmed that no significant fire has occurred on board necessitating the operation of the fixed fire extinguishing systems, or the portable fire-extinguishers since the last survey?

# CONSTRUCTION, STRENGHT AND MATERIALS

  - Design Loads

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |  |  |
| --- | --- | --- | --- | --- | --- |
| {vessel_name} |  |  |  |  |  |

B.1.1 Has the deck loading plan been inserted in the unit's operating manual?

  - Special Considerations For Column-Stabilized Units

    - Where underwater bracing is normally watertight, has the leak detection system to detect fatigue cracks examined and found satisfactory?..............................................................................................

B.3. Construction Portfolio

B.3.1 Has a copy of construction portfolio been placed on board the unit?...............................................

# SUBDIVISION, STABILITY AND FREEBOARD

C.1. Inclining test

C.1.1 Has a record of all changes to machinery, structure, outfitting, and equipment that affect the light ship date, been maintained in the operating manual or in a light ship data alterations log and been taken into account in daily operations?............................................................................................................

C.2. Watertight Integrity ……………………………………………………………………………………………………………………………

    - Is the condition of the hull and its closing appliances, as far as can be seen, satisfactorily?.............

    - Is  the  condition  of  watertight  doors,  hatch  covers,  windows,  side  scuttles,  and  portlights satisfactory?.......................................................................................................................................................

    - Were the watertight doors operational and tested (remotely or locally)?........................................

    - Is the condition of watertight bulkhead penetrations, as far as could be seen, satisfactory?...........

# MACHINERY INSTALLATIONS FOR ALL TYPES OF UNITS

  - Machinery Requirements

    - Where overspeed devices are fitted, have they been examined and found satisfactory?...............

    - Where relief valves, blow out protection devices or other means to prevent excessive pressure are fitted, were they examined and found satisfactory?..................................................................................

    - Where applicable, have crankcase explosion relief valves been examined and found satisfactory

    - Where automatic shutoff arrangements or alarms are provided, have they been tested and found satisfactory?............................................................................................................................................

    - Are means provided to ensure that normal operation of vital systems, such as ballast systems is semi-submersible units, jacking systems in self-elevating units or control of blow-out preventers can be sustained or restored, even though one of the essential auxiliaries becomes inoperable?.............................

  - Steam Boilers and Boiler Feed System

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |  |  |
| --- | --- | --- | --- | --- | --- |
| {vessel_name} |  |  |  |  |  |

D.2.1 Have safety valves been examined and found satisfactory?.............................................................

    - Have the safety arrangements and alarms been tested and found satisfactory?.......................

    - Have  the  feed  water  systems  including  the  feed  pumps  been  examined  and  found satisfactory?.......................................................................................................................................................

    - Have  the  means  for  indicating  water  level,  including  direct  reading  gauge  glass,  been examined and found satisfactory?....................................................................................................................

  - Steam Piping System

D.3.1 Has the condition of steam piping been examined and found satisfactory?................................

  - Machinery Controls

D.4.1 Has the condition of the machinery controls, including manual override been examined and found satisfactory?............................................................................................................................................

  - Air Pressure Systems

D.5.1  Have  the  air  pressure  systems  including  pressure  relieve  arrangements,  starting  air arrangements, and starting air pipes been examined and found satisfactory?................................................

  - Arrangements For Oil Fuel, Lubricating Oil And Other Flammable Oil

D.6.1 Have the oil fuel, lubricating oil, and other flammable oil arrangements been examined and found satisfactory?............................................................................................................................................

  - Bilge Pumping Arrangements

D.7.1 Have the bilge pumping arrangements been examined and found satisfactory?........................

  - Ballast Pumping Arrangements On Column Stabilized Units

D.8.1 Have the ballast pumping arrangements including air pipes, control and indicating system, and internal communications been examined and found satisfactory?...........................................................

  - Anchoring Arrangements For Surface And Column Stabilized Units

D.9.1 Has the condition of anchoring arrangements been examined and found satisfactory?............

  - Dynamic Position System

D.10.1 Has the dynamic positioning system examined and found satisfactory?.....................................

# ELECTRICAL INSTALLATIONS FOR ALL TYPES OF UNITS

  - Main Source Of Electrical Power

E.1.1 Has the main source of electrical power been examined and found satisfactory?.......................

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |  |
| --- | --- | --- | --- | --- |
| {vessel_name} |  |  |  |  |

  - Emergency Source Of Electrical Power

    - Has the emergency source of electrical power been examined and found satisfactory?..................

  - Starting Arrangements For Emergency Generators

    - Have the starting arrangements for emergency generators been examined, tested, and found satisfactory?.......................................................................................................................................................

  - Precautions Against Shock, Fire And Other Hazards of Electrical Origin

    - Are exposed metal parts of electrical machines or equipment earthed (grounded)?.......................

    - Are the sides and backs and where necessary, the fronts of switchboards suitably guarded?.........

    - Is each separate circuit protected against short circuit and against overload?.................................

    - Are accumulator batteries suitably housed and efficiently ventilated?.............................................

  - Internal Communication

    - Has the internal means of communication been examined, tested, and found satisfactory?...........

# MACHINERY AND ELECTRICAL INSTALLATIONS IN HAZARDOUS AREAS FOR ALL TYPES OF UNITS

  - Ventilation Spaces

F.1.1 Has the means of ventilation for the hazardous enclosed spaces been examined and found satisfactory?.......................................................................................................................................................

F.2 Emergency Conditions Due To Drilling Operations

F.2.1 Have the means for selective disconnection or shutdown of ventilation systems, non-essential equipment, emergency equipment, and generator prime mover been examined and is the risk of unintentional stoppage or inadvertent operation minimized?.........................................................................

# MACHINERY AND ELECTRICAL INSTALLATIONS FOR SELF-PROPELLED UNITS

  - Means Of Going Astern

G.1.1 Have the means of going astern been examined and found satisfactory and is there a record of stopping times, unit headings and distances and trial results available on board…………………………………………

  - Steam Boilers And Boiler Feed System

    - Have the high-water-level alarm and the means to prevent overpressure been tested and found satisfactory?.......................................................................................................................................................

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |  |
| --- | --- | --- | --- | --- |
| {vessel_name} |  |  |  |  |

  - Machinery Controls

    - Have the means for operation and control of the propulsion machinery been examined and found satisfactory?............................................................................................................................................

  - Steering Gear

    - Has the steering arrangements including their associated equipment and control systems been examined, tested, and found satisfactory? ………………………………………………………………………………………………….

G.4.2 Was full movement of the rudder verified?.......................................................................................

  - Communication Between The Navigating Bridge And The Engine Room

    - Have all independent means for communicating orders from the navigating bridge to machinery space from which the engines are normally controlled been examined and found satisfactory?....................

  - Engineer’s Alarm

G.6.1 Has the engineers' alarm been tested and found satisfactory?.........................................................

# PERIODICALLY UNATTENDED MACHINERY SPACES FOR ALL TYPES OF UNITS

  - Fire Safety

H.1.1 Have oil fuel and lubricating oil pipes and daily service oil fuel tanks been examined and found satisfactory?.......................................................................................................................................................

  - Protection Against Flooding

    - Have the audible and visual alarm for high bilge-water level been tested and found satisfactory?

  - Bridge Control Of Propulsion Machinery

    - Has the bridge control including means for receipt of propulsion machinery orders, indicators, and alarms been examined, tested, and found satisfactory?............................................................................

  - Communication

    - Has reliable means of vocal communication been tested and found satisfactory?...........................

  - Alarm System

    - Has the alarm system arrangement been examined, tested, and found satisfactory?.....................

  - Safety System

    - Has the safety system including automatic shutdown, alarm, and arrangements for overriding the shutdown, if fitted, been examined and found satisfactory?.....................................................................

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |  |
| --- | --- | --- | --- | --- |
| {vessel_name} |  |  |  |  |

# FIRE SAFETY

  - Protection Of Accommodation Spaces,  Service Spaces, And Control Stations

    - Have ducts, fire dampers, windows, and side scuttles facing the drill floor been examined and found satisfactory?............................................................................................................................................

  - Means Of Escape

I.2.1 Are the means of escape free of obstruction?....................................................................................

  - Fire Pumps, Fire Mains, Hydrants And Hoses

    - Has condition of fire pumps, fire mains, hydrants, hoses, nozzles, applicators, and tools been examined and found satisfactory?....................................................................................................................

    - Is each pump capable of delivering at least one jet simultaneously from each of any two fire hydrants, hoses, and 19mm nozzles while maintaining a minimum pressure of 0.35 newtons per square millimeter at any hydrant?................................................................................................................................

  - Fire Extinguishing Systems In Machinery Spaces And In Spaces Containing Fired Processes

    - Are fixed firefighting system controls, piping, instruction, and markings properly maintained and services?............................................................................................................................................................

I.4.2 Date of last reported system test……………………………………………………………………………………………………

  - Portable Fire Extinguishers In Accommodation, Service And Working Spaces

    - Are all approved portable fire extinguishers in their stowed position and fully charged?................ I.5.2 Have They been serviced within the past year?.................................................................................. I.5.3 Last serviced date ………………………………………………………………………………………………………………………….

  - Fire Detection And Alarm System

    - Has examination and test, insofar as practicable, of fire detection and alarm system and smoke detectors been carried out and all found satisfactory?.....................................................................................

  - Gas Detection And Alarm System

    - Have fixed gas detection, alarm system, and portable gas monitoring devices been examined, tested, and found satisfactory?.........................................................................................................................

  - Firemen’s Outfit

    - Are the firemen's outfits complete and in satisfactory condition?.....................................................

  - Arrangements In Machinery And Working Spaces

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

    - Have the means for stopping ventilating fans serving machinery, and working spaces for closing all doorways, ventilators, annual spaces around funnels, and other openings to such spaces been examined and found satisfactory?....................................................................................................................

  - Provisions For Helicopters Facilities

    - Are fire extinguishing arrangements for the protection of the helicopter operating area found satisfactory?.......................................................................................................................................................

    - Have the dry powder extinguishers, foam application system, carbon dioxide  extinguishers, dual purpose nozzles and hoses on the helicopter deck examined and found satisfactory?..........................

    - Has the designated area for the storage of fuel tanks been examine	and found satisfactory?..

    - Where portable fuel storage tanks are used, have they been examined and	found satisfactory?.....................................................................................................................................................

    - Have electrical fuel pumping units and associated control equipment been examined and found satisfactory?...........................................................................................................................................

    - Have procedures and precautions for refueling operations been posted?.....................................

    - Is all of the electrical bonding equipment used in refueling operations in good order?................

    - Is the unit provided with ample "no smoking" signs for use in the appropriate locations?............

  - Storage Of Gas Cylinders

    - Have the storage arrangements for gas cylinders been examined and found	satisfactory?........

    - Are fire extinguishing arrangements for the protection of areas or spaces which such cylinders are stored satisfactory?....................................................................................................................................

  - Miscellaneous Items

I.12.1 Are fire control plans posted?..........................................................................................................

I.12.2 Are means provided so that fire extinguishing appliances can be kept in good order	and	be available for immediate use at all times while engaged in drilling operations or in transit?..........................

# LIFE SAVING APPLIANCES AND EQUIPMENT

  - General

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

    - Are all life-saving appliances complying with Regulation III/30.2 of the 1974 SOLAS Convention and have they been examined and found satisfactory?.................................................................................

  - Survival Craft

    - Was a general examination of all lifeboats and life rafts and their equipment carried out insofar as practicable and all found satisfactory?........................................................................................................

J.2.2 If practicable, was one of the lifeboats lowered to the water?.........................................................

    - Were motor lifeboat engines and propulsion tested both in the ahead and astern modes?..........

    - Have inflatable life rafts been serviced during the past 12 months………………………………………………

J.2.5 Last serviced date…………………………………………………………………………………………………………………………

  - Survival Craft Muster And Embarkation Arrangements

    - Have  muster  and  embarkation  arrangements  including  emergency  lights  and  embarkation ladders been examined, tested, found, and placed in satisfactory condition?...............................................

  - Survival Craft Launching Stations

    - Have survival craft launching stations examined and found satisfactory?.........................................

  - Stowage Of Survival Craft

    - Have  all  survival  crafts  including  rescue  boat  been  stowed  in  a  seaman  like  manner,  fully equipment and ready for immediate use?.......................................................................................................

  - Survival Craft Launching And Recovery Arrangements

    - Have launching and recovery arrangements including emergency lighting, for lifeboats and davit launched life rafts been examined, exercised, and found in satisfactory condition?......................................

    - Have life raft float free arrangements been examined and found satisfactory?...............................

  - Rescue Boat

    - Has the rescue boat been examined with all required fittings, equipment, markers, engine tested in operation, been found and placed in a satisfactory condition?........................................................

  - Rescue Boat Embarkation, Launching And Recovery arrangements

    - Have rescue boat embarkation, launching and recovery arrangements, been examined and found that the rescue boat can be boarded and launched or recovered in the shortest possible time?.....

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

  - Life Jackets

    - Have  check  for  proper  stowage  of  lifejackets  and  a  random  examination  of  condition  of lifejackets been carried out?............................................................................................................................

  - Immersion Suits

    - Have immersion suits been randomly examined, found in satisfactory condition, and properly stowed?............................................................................................................................................................

  - Life Buoys

    - Are lifebuoys in satisfactory condition and are the required number fitted with; self-igniting lights, self-activating smoke signals, and are all properly placed?...................................................................

  - Radio Life Saving Appliances

    - Have the two-way VHF Radiotelephone apparatus in the lifeboats and on the unit	examined, tested, found satisfactory, and properly stowed?.............................................................................................

    - Have the radar transponders in the lifeboats and on the unit been examined, tested, found satisfactory, and properly placed?...................................................................................................................

  - Distress Flares

    - Has the unit been provided with at least twelve rocket parachute flares and properly stowed?..

J.13.2 Given expiration date…………………………………………………………………………………………………………………..

  - Line-Throwing Appliances

J.14.1 Have the line-throwing appliance found in good order?.................................................................

J.14.2 Given expiration date………………………………………………………………………………………………………………….

  - Emergency Warnings

    - Was an operational test of general alarm system carried out and found satisfactory?..................

  - Operating Instruction

    - Have illustrations and instructions been provided on or in the vicinity of survival craft and their launching controls?..........................................................................................................................................

  - Operational Readiness, Maintenance And Inspections

    - Are all lifesaving appliances maintained in working order and ready for immediate use?.............

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

    - Have the following tests and inspections been carried out weekly and recorded in the log book:

    - Have all survival crafts, rescue boats, and launching appliances been   visually	inspected	to ensure that they are ready for use?...................................................................................................................

    - Have all engines in lifeboats and rescue boats been run ahead and astern for a total period of no less than 3 mins ?........................................................................................................................................

J.17.5 Has the general alarm system been tested?...................................................................................

    - Has inspection of the life-saving appliance including lifeboat equipment and   emergency lighting been carried out monthly and has a report of the inspection been entered in the log book?.........

    - Has every inflatable life raft and inflatable lifejacket been serviced during the past 12 months?.

J.17.8 Last serviced date ………………………………………………………………………………………………………………………

J.17.9 Have the hydrostatic release units been serviced during the past 12 months?..............................

J.17.10 Last serviced date……………………………………………………………………………………………………………………..

# RADIO COMMUNICATION INSTALLATIONS

  - Self-Propelled Units Underway

    - Can the unit, while underway at sea, comply with the applicable provisions concerning radio communications for ships prescribed in Chapter IV of the 1988 SOLAS Amendments?.........................

  - Units When Towed, Self-Propelled And Accompanied By Escort Ships

    - In cases where the towing ship complies fully with all applicable requirements concerning radio communications for ships prescribed in Chapter IV of the 1988 SOLAS Amendments, does the non-self- propelled unit under tow when manned comply with the following:…………………………………………………..

    - fitted  with  VHF  facilities  as  required  by  regulations  IV/7.1.1  and  7.1.2  of  the  1988  SOLAS Amendments and with MF facilities as required by regulations IV/9.1.1 and 9.1.2?......................................

    - fitted with the satellite EPIRB or EPIRB required by regulation IV/7.1.6, as appropriate, for the area in which the MODU is being towed?........................................................................................................

    - fitted with equipment for automatic reception of navigational and meteorological warnings in accordance with regulations IV/7.1.4 and IV/7.1.5, as appropriate, of the 1988 SOLAS Amendments?........

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

    - In cases where the towing ship does not comply fully with the applicable requirements concerning radio communications for ships prescribed in Chapter IV of the 1988 SOLAS Amendments, does the unit under tow when manned comply with all the applicable provisions concerning radio communications prescribed in Chapter

IV of the 1988 SOLAS Amendments?...............................................................................................................

    - In case where self-propelled unit accompanied by one or more escort vessels, does	the	unit comply with paragraph 11.3?..........................................................................................................................

    - While stationary at the site, including when engaged in drilling operations, does the unit comply with all requirements prescribed in chapter IV of the 1988 SOLAS Amendments that are applicable to a ship sailing through the same area?................................................................................................................

    - Taking into account the different types of accidents which may occur on the MODU, additional radio equipment should be installed in a room or position, which could be the bridge or an emergency control room, situated as far as practicable from the radio equipment fitted in compliance with section 11.5.1, so that no single accident in any part of the MODU could deprive the MODU of all facilities for radiocommunications?.....................................................................................................................................

    - Does the additional radio equipment comply with the following regulations of the 1988

SOLAS Amendment for MODUs drilling in:………………………………………………………………………………………………..

K.2.9.1 sea area A1, the equipment prescribed by regulation IV/7.1.1?.............................................

      - sea area A2, the equipment prescribed by regulations IV/7.1.1 and IV/9.1.1?.......................

      - sea area A3, the equipment prescribed by regulations IV/7.1.1 and IV/10.1.1, plus 10.1.2; or alternatively, as required by regulations IV/7.1.1 and 10.2.1…………………………………………………………………

      - sea area A4, the equipment prescribed by regulations IV/7.1.1 and IV/10.2.1?.....................

    - Where the acoustic noise level in a room fitted with operating controls for radio equipment is so high or could be so high, during particular operating conditions, that it may disturb or prevent proper use of the radio equipment, has adequate noise protection been provided by mechanical or other means, in association with the operating controls for the radio equipment?................................................

  - Helicopter Communications

    - Where the unit can be serviced by helicopters, is it carrying an aeromobile VHF

radiotelephone station complying with the relevant requirements of ICAO?................................................

  - Internal Communications

    - Is the unit fitted with efficient means of communication between the control room, the bridge (if provided) and any position or positions fitted with facilities for operation of radio equipment?.............

  - Performance Standards

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

    - Are all radio equipment of a type approved by the Administration issuing the license?................

K.6. Gas Explosion Danger

K.6.1 Is any radio equipment installed in a zone as defined in 6.1 in compliance with

Regulation IV/14 of the 1988 SOLAS Amendments?.......................................................................................

  - Survey Of The Radio Station

    - Has the radio station been subject to any of the following surveys:

      - By the Administration which issues the license or its authorized representative, before the radio station is put into service?.....................................................................................................................

      - Has a survey been carried out by another coastal state if the unit has been moved and comes under the administrative control of that state?..................................................................................

K.7.1.3 Has a survey been carried out in the past 12 months?.............................................................

K.7.1.4 Give last Date……………………………………………………………………………………………………………………….

    - In every case when an authorized representative of the coastal State carries out past inspection, has a report been issued and kept with the radio documents?...................................................

# LIFTING DEVICES

  - Cranes

L.1.1 Have the cranes been examined and found in good order?............................................................

L.1.2 Has each crane been examined at intervals not exceeding 12 months?.........................................

L.1.3 Last examined date …………………………………………………………………………………………………………………….

L.1.4 A crane manual has being provided for each crane complying with the requirements of the MODU Code......................................................................................................................

  - Personnel Lifts

    - Have the personnel lifts been examined and found in good order?................................................

    - Has each personnel lift been examined at intervals not exceeding 12 months?.............................

L.2.3 Last examined date …………………………………………………………………………………………………………………….

# HELICOPTER FACILITIES

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

  - Construction

    - Have the non-skid surface and drainage facilities on the helicopter deck been examined and found in good order?......................................................................................................................................

  - Arrangements & Tied down points

    - Has the helideck safety net and Tied down points have been found in good order?..............

  - Visual Aids

    - Is the wind direction indicator, deck's marking, and deck lights in good order?............................

# OPERATING REQUIREMENTS

  - Operating Manual

N.1.1 Has the approved Operating Manual been sighted on board?.......................................................

  - Dangerous Goods

    - Have the storage arrangements for dangerous goods, explosives, flammable liquids, flammable gases, or radioactive materials been examined and found satisfactory?.......................................................

  - Diving Systems

N.3.1 Were diving systems examined and found satisfactory?................................................................

  - Safety Of Navigation

    - Does the unit comply with the requirements of the Convention on the International Regulations for Preventing Collisions at Sea in force?...................................................................................

  - Emergency Procedures

    - Has a person in charge been designated and is he well aware of the characteristics of the unit and his responsibilities?..................................................................................................................................

    - Is there a sufficient number of trained and certificated persons on board for manning of survival craft and supervision?........................................................................................................................

    - Has a muster list, with all necessary information, been verified on board?....................................

  - Emergency Instructions

    - Have illustrations and instructions been conspicuously displayed at muster stations, control positions, working spaces, and accommodation spaces to inform all on board?...........................................

  - Training Manuals

NAME OF SHIP

IMO

DATE

JOB REQUEST NUMBER

N.7.1 Is a training manual complying with Regulation III/51 of the 1974 SOLAS Convention provided, and relevant information made available to each person on board?.............................................................

  - Practice Musters And Drills

    - Has one abandon unit drill and one fire drill been conducted every week?...................................

    - Has entry into enclose Space and rescue Drills been conducted?...................................

    - Have different lifeboats, as far as practicable, been lowered at successive drills?.........................

    - Has each lifeboat, as far as practicable, been launched and maneuvered in the water  at	least once every three months?..............................................................................................................................

    - As far as is reasonably practicable, have rescue boats, including lifeboats which are also rescue boat, launched each month with their assigned crew aboard and maneuvered in the water?.....................

  - On-Board Training And Instructions

    - Have instructions in the use of unit's lifesaving appliances and in survival at	sea been given at the same interval as the drills?...................................................................................................................

    - Has on-board training in the use of davit-launched life rafts, if applicable, been taken place at intervals of no more than four months?.........................................................................................................

N.9.3 Give last date………………………………………………………………………………………………………………………………

  - Records

    - Have the date and details of musters, drills, and training been recorded in such log-book as may be prescribed by the Administration?.....................................................................................................

# RECORD OF LIFESAVING APPLIANCES

The lifesaving appliances provide for a total number of		persons and no more as follows

Totally enclosed, motor propelled, self-contained air supported and fire-protected lifeboats, on the Starboard side/forward capable of accommodating	Persons. (Reg. 10.2)

Totally enclosed, motor propelled, self-contained air supported, and fire-protected lifeboats oh the Port/aft side capable of accommodating		Persons. (Reg 10.2)

Life rafts, capable of floating and breaking free in the event of the unit becoming submerged, on the starboard side/forward and capable accommodating	 	 Persons. (Reg. 10.2)

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

Life rafts, capable of floating and breaking free in the event of the unit becoming submerged, on the Por/aft side and capable of accommodating		Persons. (Reg. 10.2)

Life raft capable of accommodating		Persons stowed as far forward/aft in addition to the life rafts provided in the above (Reg. 10.2.3)(surface unit only)

Rescue boat	Included in the total lifeboats shown above. (Req. 10.7)

Lifejackets complying with Regulation III/32.1 or III/32 of the 1974 SOLAS Convention (Req. 10-10)

Immersion suits complying with Regulation III/33 of the 1974 SOLAS Convention (Req. 10-11)

Lifebuoys complying with Regulation III/31 of the 1974 SOLAS Convention (Req. 10.12)

EITHER ATTACH OR SKETCH DIAGRAM OF LSA ARRANGEMENTS

# MODU CODE SAFETY CERTIFICATE STATUS

IMO MODU Code Safety Certificate was endorsed by the undersigned at the port of

Date:		Signed:

Conditional IMO MODU Code Safety Certificate valid until		(date) pending rectification of the following deficiencies was issued{expiry_date}

# REMARKS: (INCLUDE DEFICIENCIES, EXEMPTIONS AND ANY OTHER SITUATIONS)

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

| Issued at: |  | Date of Issue: |
| --- | --- | --- |
| {place_of_survey} | (Place of issue of certificate) | {issue_date} |

THIS IS TO CERTIFY THAT: This survey report is correct in all respects.

# GR CLASS Surveyor Name  - ID No.