| Name of ship | IMO No. | Port of Registry | Year of Built | Gross Tonnage/ Cargo Capacity |
| --- | --- | --- | --- | --- |
| {vessel_name} | {imo_number} | {port_of_registry} |  | {gross_tonnage} |

Notes:

# GR CLASS CLASSIFIED FOR STANDARD (GR CLASS)

# SUPPLEMENT TO THE REPORT OF CARGO SHIP SAFETY CONSTRUCTION SURVEY FOR RO-RO VESSELS

- Where dates are to be reported give Day, Month and Year

- Any deficiencies, repairs, recommendations, notations should be clarified under “Surveyor’s Notes” with reference by item number

- For the purpose of this form a RO-RO ship is such a ship fitted with a loading ramp which enables wheeled vehicles to be rolled-on and rolled-off the ship.

- Code Number	Y = Yes/True N = No/False

N/A = Not Applicable

- Items are applicable for all types of RO-RO ship if not mentioned otherwise.

| A. | RO-RO Passenger Ship | With bow door and keel laying before 01.07.96 or With side or stern door and keel laying before 01.07.1997 |
| --- | --- | --- |
| B. | RO-RO Passenger Ship | Other than A |
| C. | RO-RO Cargo Ship | With bow door and keel laying before 01.07.96 or With side stern door and keel laying before 01.07.1997 |
| D. | RO-RO Cargo Ship | Other than C. |

| Type of Door | Number of Doors | Location (Deck, Frame) |
| --- | --- | --- |
| Bow Visor Door |  |  |
| Bow Side Opening Door |  |  |
| Inner Door |  |  |
| Side Shell Door |  |  |
| Stern Door |  |  |
| Others |  |  |

- Survey of RO-RO facilities in accordance with class regulations to be credited for

  - Annual Survey

  - Intermediate Survey

  - Renewal Survey

  - Other Survey

- The survey of RO-RO facilities was carried out completely, with satisfactory results and may be credited for the survey indicated above.

- Parts which remain to be examined are listed in the Surveyors Note

- Alterations (see Part 1, items 1.12,1.1.3)

  - The survey showed no alterations affecting the Cargo/Passenger Ship Safety Construction Certificate

  - Alterations and/or renewals affecting the Cargo/Passenger Ship Safety Construction Certificate are listed it

  - The Supplement to the Cargo/Passenger Ship Safety Construction Certificate has been endorsed

# PART 1 – DOCUMENTS

  - OPERATING AND MAINTENANCE MANUAL (OMM) (mandatory for type A,B and D) The OMM contains information on main particulars, design drawings, acceptable clearance for supports, service conditions and service restrictions, maintenance and function testing, register of inspection and repairs.

    - Is the copy approved by GR?	Date of approval

    - There are no additions/modifications in the manual, which are not approved by GR?

    - No modifications or damage have been reported to the surveyor?

    - Is the OMM endorsed for inspections of the door supporting and securing device by the ship’s staff at	a) monthly intervals

and	b) following incidents that could result in damages (e.g. heavy weather or contact in the region of the shell door)

    - The OMM is endorsed by the Surveyor after having performed the survey.

  - OPERATING PROCEDURES (mandatory for type A, B and D)

Are documented operating procedures for closing and securing the doors kept On board and posted at an appropriated place?

VENTILATION SYSTEM (Chapter II-2, Reg. 16)

2.1.1	Are ventilation ducts for machinery spaces of category A, galleys, car deck spaces and ro/ro cargo spaces passing through accommodation spaces, service spaces or control stations?

2.1.1 If water spraying system is fitted in ro/ro cargo  spaces,  are  sufficient  drainage  and pumping arrangement to prevent the build-up of free surfaces fitted? (Chapter II-2, Reg. 53)

Are at least two widely separated escapes provided from ro/ro cargo spaces where crew is normally employed?

RO/RO CARGO SPACES - MOTOR VEHICLES WITH FUELIN THEIR TANKS

(Chapter II-2, Reg.  53.2.3)

- PART 2 – REQUIRED VISUAL EXAMINATION

## STRUCTURAL ARRANGEMENT

- Cracks are not accepted as permanent; cracks found in vital elements, such as supporting structure, supports, lifting arms, etc. will have to be repaired immediately or, if in other areas and not critical, to be given a very short time repair limit together with possible operational restriction.

- Whenever a crack is found in a vital element, a through examination with NDT is to be carried out in the surrounding area and for similar items (i.e. other supporting devices).

- NDT of vital elements and thickness measurement may be required by the Surveyor after visual examination.

    - Are the following structural elements and their welding found to be in sound condition And free of corrosion, deformation and any damage?

      - Plating (To be treated as shell plating)

      - Primary structure

(Longitudinal and transverse girders and frames; When the overall strength of the Member is affected by any findings, repairs or reinforcements are recommended)

      - Secondary stiffeners

Definition:

Securing device	a device used to keep the door closed by preventing it from rotating about its hinges.

Supporting devices    a device used to transmit external or internal loads from the door to a securing device and from the securing device to th4e ships structure, or a device other than a securing device, such as a hinge, stopper or other fixed device, that transmit loads from the door to the ships structure

Locking device	a device that locks a securing device in the closed position

        - Required for doors having the sill below the uppermost load line.

        - Required only for doors with clear opening area >=12m2

        - Required for doors which may lead to the flooding of a special category space or ro-ro cargo space.

Where no part of the door is below the uppermost waterline and the are of the door opening is not greater then 6 m2, the requirement needs not to be applied.

      - Shell plating surrounding the opening and the securing, supporting and locking devices

      - Hinging arms, supports for securing, supporting and locking devices on hull and door sides

(To be treated as primary structure. Visible deformation are not accepted. The maximum Allowable margin of diminution is 15% of the original thickness.)

## Hinges, bearing, thrust bearing

      - Is the clearance of hinges, bearing, thrust bearings taken where no dismantling was necessary? (Not applicable for Renewal Survey)

      - Is the clearance of hinges, bearing, thrust bearing taken for class renewal or where The function test is not satisfactory?

(Maximum clearance is given in the OMM according to the Manufacturer’s

specification, if provided, or the design value as per the OMM plus 2 mm.)

      - Is NDT performed on hinges, bearing, thrust bearing and on the hydraulic opening/securing/looking devices?

(Not applicable for Annual Survey)

## SECURING, SUPPORTING AND LOCKING DEVICES

    - Are the securing, supporting and locking devices including their welding examined by a Close visual inspection?

    - Are the clearances measured, where indicated in the OMM?

    - Is a close visual inspection carried out, supplemented by NDT and thickness measurement? (Not applicable for Annual and Intermediate Survey)

    - Are the clearances within the required margin?

(Not applicable for Annual and Intermediate Survey)

(Maximum clearance is given in the OMM according to the Manufacturer’s specifications, if provided, or the design value as per the OMM plus 2 mm).

In case of findings the following is to be observed:

    - Is NDT at primary welds and elsewhere carried out?

    - No cracks or deformations have been detected?

    - Has a through examination been carried out, supported by NDT in the surrounding Area and for comparable devices where cracks have been revealed?

    - Are thickness measurements carried out as deemed necessary by the surveyor after visual inspection

    - Are the results of the thickness measurements within the required margin? (Maximum diminution is 15% of the original thickness)

    - Is a visual inspection of all cylinders carried out?

    - Is a visual inspection of the hydraulic piping system carried out?

## PACKING MATERIAL/RUBBER GASKETS

    - Has a close visual inspection been performed?

    - Is the permanent compression of seals within the allowable margin?

(The maximum value is given in the OMM according to the Manufacturer’s Specifications, if provided, or 10% of the packing thickness)

    - Have packing been renewed which were damaged due to ageing / hardening?

(Material to be of the same type as the original as far a possible, otherwise it should have equivalent characteristics).

    - Are the packing free of painting

    - Have the packing retaining bars or channels, including welding been examined?

    - Have repairs of the packing retaining bars or channels, including their welding been recommended? (Repairs have to be recommended whenever the efficiency of the packing may be

Impaired by wasted areas, cracks and deformations).

  - DRAINAGE ARRANGEMENT (applicable for type A, B and D where installed (1))

    - Have the bilge wells been examined?

    - Have the drain pipes been examined?

    - Have the non return valves been examined internally? (Not applicable for Annual and Intermediate Survey)

## PART 3 – REQUIRED FUNCTION TESTS

  - Shell and inner doors (applicable where installed)

    - Has the door smooth symmetrical movement?

    - Do the hinging arms and hinges work properly?

    - Are the thrust bearings engaged properly?

    - Is the device for locking the door in the open position working properly?

## Securing, supporting and locking devices

    - Have all the devices been examined during a complete opening and closing operation and checked According to item 3.2.2 – 3.2.9?

    - Have the devices run smoothly without interference with surrounding structure?

    - Are the opening/closing system and the securing/locking devices interlocked in way that they can Only operate in proper sequence? (Applicable for type A, B and D)

    - Is the mechanical lock of the securing devices working properly?

    - Does the hydraulic securing devices remain locked according to the procedure provide by the OMM in the event of a loss of hydraulic fluid?

    - Are there correct indications of open / closed position of doors and securing / locking devices at the Remote control stations, wheelhouse included?

    - Can the hydraulic securing/locking devices be safety isolated from other hydraulic systems? (applicable for type B and D)

    - Are the operating panels inaccessible to unauthorized persons? (Applicable for B and D; applicable for type and C (2))

    - Is a notice plate giving instructions to the effects that all securing devices are to be closed and locked

Before leaving harbour? Is this plate placed at each operating panel and supplemented by warning indicator lights?

## Sealing arrangement

    - Has a tightness hose test or equivalent been performed with satisfactory result?

## Indicator system

GR CLASS – CLASSIFIED FOR STANDARD (GR CLASS)
                                                                                                                     E-mail: info@grclass.com Web: www.grclass.com

Form CCC-05	                                                                                   Approved by: GM

Page 4 of 5

    - Has the indicator system been examined and tested according to item 3.4.2 – 3.4.7?

    - Have the visible indication and audible alarm on the navigation bridge panel, according to the

Selected function “harbour / sea voyage” and on the operating panel been tested for proper function? (applicable for type A (where installed), B and D (3))

    - Have the lamps on both panels been tested for proper function?

    - Has it been tested that it is not possible to turn off the indicator light on both panels?

    - Has the fail safe performance been tested, according to the procedure provided by the OMM?

    - Has it been tested that power supply for indicator system is independent of the power supply for Operating the doors and supplied by the emergency source?

    - Are the sensors and their protection from water, ice formation and mechanical damage still in Effective condition and have they been tested?

## Water leakage detection system:

    - Has working test been performed?

    - Have the audible alarm on the navigation bridge panel and o the engine control room panel been Checked for proper working according to the procedure provided by the OMM?

  - Television surveillance system (applicable where installed)

    - Has working test been performed?

    - Have the proper indication on the navigation bridge monitor and on the engine control room Monitor been checked?

## Electrical equipment

    - Have the equipment for opening, closing and securing the doors been examined?

## DRAINAGE SYSTEM

    - Has a test of the bilge system between the inner and outer doors been carried out?

## Place and Date of Survey	GR  CLASS Surveyor Name – ID No.

GR CLASS – CLASSIFIED FOR STANDARD (GR CLASS)
E-mail: info@grclass.com Web: www.grclass.com

Form CCC-05	                                                                         Approved by: GM	 	                                                                                  Page 5 of 5