# GR CLASS CLASSIFIED FOR STANDARD (GR CLASS)

Table D

(The following requirements govern the application of Table 2.5) Documents/information to be submitted for

COAL/ BROWN COAL BRIQUETTES

| Name of ship | Distinctive number or letters | Port of registry | Gross Tonnage |
| --- | --- | --- | --- |
| {vessel_name} | {call_sign} | {port_of_registry} | {gross_tonnage} |
|  | IMO Number: |  |  |

| Requirements on Table 2.3 | Documents/information to be submitted The meaning of “L” is specified under this table |  |
| --- | --- | --- |
| Boundaries of cargo spaces should be resistant to fire and liquids. | ---- | ----------- |
| Electrical cables and components situated in cargo spaces and adjacent spaces should be free from defects and safe for use in explosive atmosphere or positively isolated. | L | Arrangement and wiring diagram of electrical equipment fitted in the space including grade of each equipment, such as IIAT4. |
| Appropriate instruments for measuring followings into cargo spaces without entry into such spaces should be provided. Methane Oxygen Carbon monoxide pH value Temperature( 0 - 100ºC) | L | Type, manufacturer and specifications |
| Two sets of self-contained breathing apparatus to be provided. | L | Type, manufacturer and specifications |
| “No Smoking" sign and "No naked flames" sign should be posted in conspicuous places. | L | Number and locations of the signs |
| Natural surface ventilation should be provided for cargo spaces. | L | Drawings of the ventilation systems Arrangement of air holes |
| Natural or mechanical ventilation should be provided for enclosed working spaces, such as store rooms, carpenter's shops, passage ways, tunnels. Mechanical ventilation, if used, should be of safe type for use in explosive atmosphere. | L | Drawings of the system |
| Two sampling holes per hold, one on each side of the hatch cover should be provided with threaded stub and sealing cap. | L | Drawings of the system |

L: To be submitted to the local office for their checking.

Issued at	On the

GR CLASS – CLASSIFIED FOR STANDARD (GR CLASS)
E-mail: info@grclass.com Web: www.grclass.com

Form IMSBC04	                                                                     Approved by: GM	 	                                        Page 1 of 15