# GR CLASS CLASSIFIED FOR STANDARD (GR CLASS)

SURVEY REPORT FOR INTERNATIONAL OIL POLLUTION PREVENTION CERTIFICATE

Under the provisions of Annex I of the International Convention for the Prevention of Pollution from Ships, 1973, as modified by the Protocol of 1978 relating thereto, as

amended

| Initial Survey | Annual Survey (Periodical Survey) | Renewal Survey |
| --- | --- | --- |
| Intermediate Survey | Additional Survey (occasional) |  |

| Name of Ship | Distinctive Number or Letters | Port of Registry | Gross Tonnage | IMO number |
| --- | --- | --- | --- | --- |
| {vessel_name} | {call_sign} | {port_of_registry} | {gross_tonnage} | {imo_number} |
| Type of Ship | Deadweight (for Oil Tankers, Chemical Tankers and Gas Carriers only) | Date of Building Contract | Date on which keel was laid | Date of Delivery |
| {ship_type} | {deadweight} | {year_built} | {year_built} |  |

Owner / Operators1:	 			 Date of Survey / commenced:

completed:         	 Name /                   Number                   of                   Surveyor:    	 Place of Survey:

1 Delete as appropriate

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

# Note:	Mark in box

Fit or Yes

Unfit or No

Not Applicable

X

--

N/A

GENERAL

  - Alterations have been made in the construction and equipment of the ship covered by the Supplement to the International Oil Pollution Prevention Certificate ……………

  - Alterations in the construction and equipment are listed in C.1 …..……………………….

# DOCUMENTATION

  - Validity of the existing International Oil Pollution Prevention Certificate:

  - The existing International Oil Pollution Prevention Certificate has been endorsed …

  - Date of the last endorsement:

  - Validity of the existing Exemption Certificates:

  - Supplement to the International Oil Pollution Prevention Certificate (Form A or

Form B) have been issued……………………………………………………...………………………………..

  - The Oil Record Book (Part I or Part II) complies with the requirements of Regulation

17 of And 36 Annex I of the Convention and entries were duly made ….………………...

  - Dedicated Clean Ballast Tank (CBT) Operation Manual (Approved by Administration) (Reg. 18.8.4 ) (If Apply, copy of the Approval Manual to be

Attached) ……………………………………………………………………………………………………………….

  - Crude Oil Washing (COW) Operations and Equipment Manual (Approved by Administration) (Reg. 35.1) (If Apply, copy of the Approval Manual to be attached)

  - Oil  Discharge  Monitoring  and  Control  System  (ODMC)  Operational  Manual (Approved by Administration) (Reg. 31.4 ) (If Apply, copy of the Approval Manual  to be Attached)……………………………………………………………………………………………………….

  - Shipboard Oil Pollution Emergency Plan (SOPEP) approved by administration (Reg.

37) (Copy of SOPEP Approval to be attached) ……………………………………………………...

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

  - Shipboard Marine Pollution Emergency Plan (SMPEP) for Noxious Liquid Substances (Reg. 37) Approved by Administration (If Apply, copy of the SMPEP

Approval to be attached)……………………………………….………………………………………..………

  - The ship comply with the requirements of Reg. 19 …………………………………….…………..

  - Oil tanker not meeting the requirements as defined  in Regulation 1.28.5  shall comply with the requirements of Regulation 19 not later than 25 years after its date of delivery; State the date when the ship will have to comply with the requirements of Reg. 19:

  - Certificate of Type Test / Certificate of Type Approval2 for:

    - Oil Filtering Equipment (15 ppm Equipment) (Reg. 14.6) without alarm and automatic stopping device………………………………………………………………………………………. No.	(If Apply, copy to be attached)

    - Oil Filtering Equipment (15 ppm Equipment) (Reg. 14.7) with alarm and automatic stopping device (if applicable, fill out Item C)…………………………………….…… No.	(If Apply, copy to be attached)

B.14.3 Oil Content Meters (15 ppm alarm) (Reg..31.2) ……………………………………………

No.	(If Apply, copy to be attached)

B.14.4 Process Units for attachment to existing Oily-Water Separating Equipment installed on board a ship on or before 20 January 1979 (Resolution A. 444 (XI) ……… No.	(If Apply, copy to be attached)

# EQUIPMENT  FOR  CONTROL  OF  OIL  DISCHARGE  FROM  MACHINERY  SPACES  BILGES (REG. 13, 14, 15).

  - Oil Filtering Equipment (15 ppm) or Process Unit:

    - Type of equipment fitted (brand & model):

C.1.2 Test of the equipment ………………………………………………….………………………..………

C.1.3 Type of alarm fitted:

C.1.4 Test of the Alarm ……………………………………………………………………………………………

C.1.5 Automatic Stopping Device required for discharging in special areas and for

ships of 10000 tons gross tonnage and above …………………………………………………………

  - Oily bilge water holding tank on ships (>400 GT) for collecting oily bilge water prior to its discharge, transfer or disposal …………………….………………………………………………..

2 Delete as apropriate

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

  - Oil residue (sludge) tank on ships (>400 GT) from which sludge is being disposed through the standard discharge connection or any other approved means of Disposal………………………………………………………………………………………………………….……….

  - The remote cut-off of the bilge water discharge pump (s) from the area of the Standard  Discharge  Connection  (referred  in  Reg.  13);  or  the  communication (telephone or radio)between the discharge observation position and the control position of the bilge water discharge pump(s) …………………………..…………………………..

C.5	Standard Discharge Connection……………………………………………………….………………………

# SEGREGATION OF OIL AND WATER BALLAST AND CARRIAGE OF OIL IN FORE PEAK TANKS (REG. 16)

  - If the vessel carries under normal condition ballast water in oil fuel tanks …..……….

  - Tanks for oil residues (sludge) (Reg. 12)

    - The ship is provided with oil residues (sludge) tank (s) ……….………………………..

    - Designated pump for the discharge of the tank(s) content to reception

facilities …………………………………………………………………………………………………………………

    - No discharge connections to the bilge system, oily bilge water holding tank, tank top  or oily water separators, unless the tank is fitted with drains, with self-

closing valves and visual monitoring of settled water………………………………………………..

    - Homogenizers, sludge incinerators or other recognized and approved means for the destruction / disposal3 of oil residues (sludge) ………………………………………..…

    - Test of Homogenizers, sludge incinerators or other recognized and approved means for the destruction / disposal3   of oil residues (sludge) ………………………………

| D.3 | Total capacity of fuel tanks | m3 or tons |
| --- | --- | --- |
|  | D.3.1 Total capacity for heavy fuel | m3 or tons |

D.3.2 Total capacity for diesel oil	m3 or tons

# ADDITIONAL REQUIREMENTS FOR OIL TANKERS

  - Is the ship provided with Material Safety Data Sheet (MSDS) prior the loading of

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

Annex I cargoes and marine fuel oils?..........................................................................

  - Certificate of Type Test / Certificate of Type Approval3 for:

    - Oil Content Meters intended for monitoring the discharge of oil-contaminated water from the cargo tank areas of oil tankers built on or after 2

October 1986 (Reg. 31 and IMO Resolution A. 586(14) ) ……………………………………….

No.	(If Apply, copy to be attached)

    - Oil / Water interface detectors for use in slop tanks and other tanks (Reg. 32 and IMO Resolution MEPC.5 (XIII) ) ………………………………………………………………………… No.	(If Apply, copy to be attached)

  - Approved Operational Procedures for the existing oil tankers with special ballast

arrangements (Reg. 18.10)…………..………………………………………………………………………….

  - Operations Manual for the part flow system for control of overboard discharge

(Reg.30.6.5.2 )……………………………………………………………………………………….……….……...

  - Approved loading and stability information (Reg. 28)………………………………………….

  - If oil tanker >150 GT involved in ship to ship (STS) operations is a STS operation

plan approved by the administration on board?…………………………………………………….

# SEGREGATED BALLAST TANK (SBT) (REG. 18)

    - Arrangement of pumps, pipes and valves for Segregated Ballast System4 …..…

    - No cross-connections between the Segregated Ballast System and any other

System…………………………………………………………………………………..….…………………………….

    - Non-return valves on the segregated ballast connections to the cargo pump and portable spool piece mounted in a conspicuous position in the pump room with a permanent notice restricting its use (where such a portable spool piece is provided for emergency discharge of the segregated ballast by means of a

connection of Segregated Ballast System to a cargo pump)...………………………………..

E.7.4 No sign of oil contamination in the SBT ………………………………………………………….

E.7.5 No leakage from those ballast pipelines passing through cargo tanks and those cargo pipelines passing through ballast tanks5 ……………………………………………...

4 At the periodical (renewal) survey

5 At the intermediate and periodical (renewal) survey

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

# E. 8  DEDICATED CLEAN BALLAST TANKS (CBT) (REG. 18.8)

E.8.1 Arrangement of pumps, pipes and valves for clean ballast system4 ………………

E.8.2 No sign of oil contamination in the dedicated CBT …………………………………………

    - No leakage from those ballast pipelines passing through cargo tanks and those cargo pipelines passing through ballast tank5

    - Oil content meter for monitoring of the clean ballast discharge:

E.8.4.1 Type of Oil Content Meter fitted:

E.8.4.2 Test of the Oil Content Meter …………………………………………………………….

# E. 9 CRUDE OIL WASHING SYSTEM (COW) (REG. 33)

E. 9.1 No sign of leakage on COW pipelines, pumps, valves and deck-mounted

washing machines …………………………………………………………………………………………………..

E. 9.2 Fasteners of COW pipelines …………………………………………………………………………..

    - Pressure test of the COW system6

      - Test Pressure

      - Working pressure

E. 9.4 Numbers of set (s) of operational driving units of washing machines as specified in the manual (where driving units are disintegrable and portable):

    - Internal equipment and arrangements of tank6

    - Double stop valves or clearly identifiable, blind blanks for the shut-off of the

steam heaters for water washing during crude oil washing……………………………………..

    - Means of communication between the deck watchkeeper and cargo control

room……………………………………………………………………………………………………………………….

    - Safety overpressure valves (or other approved devices) fitted to the supply

pumps of the COW system………………………………………………………………………………………

    - Flexible hoses for supply of oil to the washing machines on combination

carriers……………………………………………………………………………………………………………………

    - Effectiveness of the COW System6:

6 At the periodical (renewal) survey

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

E.9.10.1 Function of the washing machines for COW (by external indicators

or sound pattern) …………………………………………………………………………………………

E.9.10.2 Function of the stripping system ……………………………………………………

E.9.10.3 Pressure in the COW line during washing	MPa

(psi multiply by 0.006894757 to obtain N/mm2 (MPa) )

E.9.11 Function of the Inert Gas System ………………………………………………………………….

# E. 10 EXISTING OIL TANKERS HAVING SPECIAL BALLAST ARRANGEMENTS (REG. 18.10)

E.10.1 Approved arrangement for the special ballasting …………………………………………

# PREVENTION OF OIL POLLUTION IN THE EVENT OF COLLISION OR STRANDING (REG.19)

    - urvey in accordance with Reg. 19.5 and 23.3……………………………………………….

# RETENTION OF OIL ON BOARD (REG. 29, 31 & 32)

    - tanks and the associated piping arrangements ……………………………………..

    - rangement for machinery space bilge discharge into slop tanks………………..

E.12.3 Oil Discharge Monitoring and control system:

E.12.3.1 Oil Content Meter …………………………..……………………………………………

E.12.3.2 Type of the Oil Content Meter fitted

      - Test of the Oil Content Meter ………………………………………………………

      - utomatic and manual devices for stopping the discharge…………… E.12.3.5 Starting interlock …………………………………………………………………………. E.12.3.6 Flowmeter ……………………………………………………………………………………

      - ndicating and recording devices ……………………………………………….…

      - Consumables for recording devices ………………………………………………

E.12.3.9 Visible and audible alarm ……………………………………………………..………

E.12.4 Oil / water interface detectors ……………………………………………………………………..

# 13 PUMPS, PIPING AND DISCHARGE ARRANGEMENTS (REG. 30)

    - Piping arrangements for discharge of dirty ballast water or oil-contaminated

Water………………………………………………………………………………………………………………………

    - Means of communication between the observation position and the discharge control position ………………………………………………………………………………………

    - Means to drain all cargo pumps and all oil lines including the stripping

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

devices and line for discharge to the slop or cargo tank or to reception facilities…….

E.13.4 Arrangement of the part flow system……………………………………………………………

# E.14 ARRANGEMENT OF CARGO TANKS (REG. 26)

E.14.1 Cargo transfer system and closing devices provided in the cargo oil pipelines For Separating the tanks from each other7………………………………………………………………

- ADDITIONAL REQUIREMENTS FOR INTERMEDIATE SURVEY

  - Were the requirements for the IOPP MAS satisfactorily completed?

  - Was the oily-water separating equipment or oil filtering equipment or process unit, where fitted,	9, 10, 16 including associated pumps, piping, and fittings examined for wear and corrosion and found satisfactory? ……………………………………………………………………

  - Was the oil content meter (15 ppm alarm and bilge monitor) checked for obvious defects, 16 deterioration or damage and found satisfactory? ……………………………………..

  - If the oil content meter is capable of being calibrated, was any record of calibration

checked against the manufacturer’s operation and instruction manual ………………………..

  - For crude oil washing systems, did the survey also include the following (In addition to item E9 of this checklist)………………..…………………………………………………………………………..

    - was crude oil washing piping outside the cargo tanks examined and found satisfactory?........................................................................................................................

    - were the isolation valves for steam heaters for water washing, when fitted, tested to ensure they are operating satisfactorily? …………………………………………………………

    - were at least two selected cargo tanks examined for the express purpose of verifying the continued effectiveness of the COW and stripping systems and found satisfactory? ………………………………………………………………………………………………………………….

  - In addition to item III.7 of the MAS, was the oil discharge monitoring and control system 15(3)(a)and oil content meter for cargo spaces examined for obvious defects, deterioration, or damage and found satisfactory? ……………………………………………………….

  - Was the manual and/or remote operation of the individual tank valves (or other similar devices) that are required to be kept closed at sea in accordance with Regulation 24 tested and found satisfactory?	24(5), (6)

7 At the periodical (renewal) survey

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

F.8 Was the satisfactory operation of the oil/water interface detectors confirmed? 15(3)(b)

Was the IOPP Certificate endorsed? ……………………………………………………………………………

Was it confirmed that no unproved modifications have been made to the ship or its equipment? …………………………………………………………………………………………………………………….. If major/minor outstanding deficiencies were found, was survey procedure, Statutory Surveys, Survey Statement Report, referred to for special instructions? ………………………….

# ADDITIONAL REQUIREMENTS FOR RENEWAL SURVEY

  - Were the applicable requirements for the MAS satisfactorily completed? ………………..

  - Were the applicable requirements for the INT Survey satisfactorily completed? ……..

  - Instead of items III. 11 and 12 of the MAS and item 5 of the INT Survey for COW : 13B systems, were the following verified …………………………………………………………….

    - were the COW piping, pumps, valves, and deck mounted washing machines examined for signs of leakage and found satisfactory ……………………………………………………..

    - were all anchoring devices for COW piping examined to ensure they are intact and secure? ……………………………………………………………………………………………………………………..

    - was COW piping pressure tested to at least the working pressure and found satisfactory? …………………………………………………………………………………………………………………..

    - as it confirmed in those cases where drive units are not integral with the tank washing machines, that the number of operational drive units as specified in the COW Manual are on board? ……………………………………………………………………………………………………

G.3.5 was it confirmed, by completing internal tank inspection, that the internal equipment and arrangements remain satisfactory? ………………………………………………………

    - were steam heaters for water washing, when fitted, checked to ensure they can be properly isolated during COW operations, either by double shut-off valves or by clearly identifiable blanks and found satisfactory? ………………………………………………………..

    - when isolation valves are fitted, were they disassembled, internally examined, and found satisfactory? ………………………………………………………………………………………………….

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

    - was the prescribed means of communications between the deck watch keeper and the cargo control position checked to confirm it is operational? ………………………………

    - was it confirmed that an overpressure relief device is fitted to the pumps supplying the COW system and in satisfactory condition? ………………………………………………..

    - was it confirmed that the flexible hoses for supply of oil to the washing machines on combination carriers are of an approved type, are properly stored, and are in satisfactory condition? ………………………………………………………………………………………………….

    - Was it confirmed that the COW system is installed in accordance with the Revised   Res. A.446(XI) & Specifications for the Design, Operation and Control of COW and was it verified Res. A.497(XII) that no unapproved modifications have been made?

    - was the effectiveness of the COW system verified in accordance with paragraph Res. A.446(XI) 4.2.10 of the Revised COW Specifications and found  satisfactory. In particular, and were the following verified if possible and practicable13: Res. A.497(XII) …………………………………………………………………………………………………………………..

      - checked the system to confirm the COW machines are operable and to observe the proper operation of the washing machines by means of the movement indicators and/or sound patterns or other approved means? ………………………………………….

      - was the efficiency of the stripping system confirmed by:  ……………………..

        - observing the monitoring equipment; and ……………………………………….

        - observing the hand dipping of the tanks?  ……………………………………….

      - was a measurement of oil on top of departure ballast made, and was

……………………………….the ratio of the volume of oil to the tank volumes below 0.00085?

      - was the arrival ballast discharged through an approved monitoring system, and was the concentration of oil in the arrival ballast not in excess of 15 ppm?

      - were at least two cargo tanks internally examined after crude oil washing to confirm that the installation and operational procedures laid down in the Operations and Equipment Manual are satisfactory? ……………………………………………………..

# CARRIAGE AND DISCHARGE OF OIL-LIKE SUBSTANCES FALLING UNDER CATEGORY C OR D CARRIED ON OIL TANKER AS DEFINED IN ANNEX I

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

  - Does the vessel comply with the provisions of Annex I of the present Convention as applicable to product carriers as defined in that Annex? ………………………………………………….

  - Does the ship carry a valid International Oil pollution Certificate and its Supplement B? ……………………………………………………………………………………………………………………………………..

  - For Category C does the ship comply with the ship type 3 damage stability requirements of ………………………………………………………………………………………………………………..

    - the International Bulk Chemical Code for ships constructed on or after 1 July 1986; or ……………………………………………………………………………………………………………………………

    - the Bulk Chemical Code as applicable under Regulation 13 of this Annex, in the case of a ship constructed before 1 July 1986? ………………………………………………………………..

H.4. Is the oil content meter in the oil discharge monitoring and control system of the ship approved by the Administration for use in monitoring the oil-like substances? ……

Was it confirmed that no unproved modifications have been made to the ship or its equipment? ………………………………………………………………………………………………………………….. If major or minor outstanding deficiencies were found, did the surveyor refer to survey procedure, Statutory Surveys, Survey Statement Report for proper instructions? …………

# IOPP CERTIFICATE: FULL TERM/ INTERIM/ CONDITIONAL/ ENDORSED ISSUED UNTIL

PENDING:

Remarks Section:

# I.	APPENDIX

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

GUIDELINES FOR FUNCTIONAL TEST OF THE OIL DISCHARGE MONITORING AND CONTROL (ODMC) SYSTEM.

The functional test referred to in paragraphs 8.1.8 and 11 of IMO Resolution A.586 (14) should include all the following tests when the monitoring system is operating on water (indicate YES, NO, N/A

I.1 Checking correct running of the pumps, absence of leakage in the sample pumping and piping system, correct functioning of remote controlled sampling valves, etc………….

  - Checking, by flow rates or pressure drops, that the system operates under correct flow conditions. This test should be repeated separately for each sampling point …………

  - Checking that alarms function correctly when a malfunction occurs external to the monitoring system such as no sample flow, no flow meter signal, power failure, etc …….

  - Checking the recordings for correct values and timing by varying the simulated input signals manually; checking proper recordings by varying the manual input signals until alarm conditions are obtained; and for category A monitoring system, ascertain that the overboard discharge control is activating, and check that the action is being recorded….

  - Checking that normal operating condition can be reset when the value of instantaneous rate of discharge is reduced below 30 liters per nautical mile………………….

  - Checking that a recording is made when the manual override control is activated; and that for a category A monitoring system, the overboard discharge control can be operated………………………………………………………………………………………………………………………….

  - Turning off the system, and for a category A monitoring system, checking that the overboard discharge control cannot be operated…………………………………………………………..

  - Starting up the system, and checking the zero gain setting for the oil content meter in accordance with the manufacturer's operations and technical manual…………………………….

  - Checking the accuracy of any installed flow meter, for example, by pumping water in a loop where the flow rate may be calculated from the level change in a tank. The check should be made at a flow rate of about 50% of the rated flow of the flow meter……………

# ALTERATIONS OR MODIFICATIONS:

F.1 Describe in detail all system and equipment or parts which have been altered

| NAME OF SHIP | IMO | DATE | JOB REQUEST NUMBER |
| --- | --- | --- | --- |
| {vessel_name} |  |  |  |

or modified...................................................................................................................

# DEFICIENCIES, REQUIREMENTS, REMARKS, RECOMMENDATIONS.

- FINAL RECOMMENDATION:

It is recommended that:

  - A Full Term IOPP Certificate may be issued valid for a period of five (5) years……………

  - No Full Term IOPP Certificate to be issued until deficiencies are closed (marked in

Item G)…………………………………………………………………………………………………………………….

  - The Existing IOPP Certificate No.	has been endorsed at:

(Place of issue of certificate)	Date of issue (DD-MM-YYYY)

  - I have issued the Interim/ Conditional8   IOPP Certificate  No.:

(Place of issue of certificate)	Date of issue (DD-MM-YYYY)

THIS IS TO CERTIFY THAT: This survey report is correct in all respects.

Issued at		Date of issue

(Place of issue of certificate)	(DD-MM-YYYY)

# GR CLASS Surveyor Name   - ID No.

8 Delete as appropriate